# I-40 Rotor Written Specifications

Part 1 – General

  - The I-40 is a gear-driven rotary sprinkler designed for commercial, municipal, and professional sports turf applications. It shall be available in full-circle opposing nozzle (ON) and adjustable part-circle to full-circle (50° to 360°) non-reversing configurations with versions tailored for conventional use and high-speed operation.

1.2 Key features shall include:

- Non-strippable drive mechanism

- Automatic arc return

- Self-adjusting stator for consistent rotation speed

- Factory-installed rubber cover with optional purple Alcryn® cover for reclaimed water identification

- Standard stainless steel riser for improved abrasion resistance

- Drain Check Valve with 15' (4.5 m) check height

Part 2 – Parts and Materials

2.1 The rotor shall be available in the following options, all of which can be ordered with a purple recycled rubber cover by adding “-R” to the end of the part number:

- I-40-04-SS: 4" (10 cm) pop-up with stainless steel riser

- I-40-04-SS-HS: 4" (10 cm) pop-up with stainless steel riser and high-speed gearing

- I-40-04-SS-ON: 4" (10 cm) pop-up with stainless steel riser and dual opposing nozzles

- I-40-04-SS-ON-HS: 4" (10 cm) pop-up with stainless steel riser, dual opposing nozzles, and high-speed gearing

- I-40-06-SS: 6" (15 cm) pop-up with stainless steel riser

- I-40-06-SS-HS: 6" (15 cm) pop-up with stainless steel riser and high-speed gearing

- I-40-04-SS-ON: 6" (15 cm) pop-up with stainless steel riser and dual opposing nozzles

- I-40-06-SS-ON-HS: 6" (15 cm) pop-up with stainless steel riser, dual opposing nozzles, and high-speed gearing

2.2 Warranty

- The rotor shall be installed in accordance with the manufacturer’s published instructions. The rotor shall carry a conditional 5-year exchange warranty. The rotor shall be the I-40 Rotor, as manufactured for Hunter Industries Incorporated, San Marcos, California.

2.3 Construction

- Heavy-duty, corrosion- and impact-resistant ABS body

- Stainless steel riser and spring

- Rubber cover (standard or purple Alcryn)

- Protective rubber boot (when retracted)

- Optional Turf Cup Kit for natural grass on top of riser

2.4 Inlet and dimensions

- The rotor shall have a standard 1" (25 mm) FNPT inlet with an optional BSP threaded inlet.

- The rotor shall have an exposed surface diameter of 2" (5 cm).

- The rotor shall have an overall height of 7⅞" (20 cm).

- The rotor shall have a pop-up height of 4" (10 cm) or 6" (15 cm), depending on model.

Part 3 – Function and Operation

3.1 Pressure and flow

- The rotor shall have an operating pressure range of 40 to 90 PSI (2.5 to 7.0 bar; 250 to 700 kPa).

- The Drain Check Valve shall hold up to 15' (4.5 m) of elevation.

3.2 Radius and flow ranges

- Conventional models:

  - The rotor shall have a radius of 44' to 69' (13.1 to 21.3 m).

  - The rotor shall have a flow rate of 7.6 to 29.5 GPM (1.6 to 6.8 m³/hr; 27.2 to 114.1 l/min).

  - The rotor shall complete a full 360° rotation in ~3 minutes.

- Opposing nozzle (ON) models:

  - The rotor shall have a radius of 52' to 76' (15.2 to 23.2 m).

  - The rotor shall have a flow rate of 13.0 to 33.7 GPM (2.8 to 7.7 m³/hr; 45.8 to 128.8 l/min).

  - The rotor shall complete a full 360° rotation in ~3 minutes.

- High-speed models:

  - The rotor shall have a radius of 41' to 63' (14.3 to 19.5 m).

  - The rotor shall have a flow rate of 7.6 to 28.5 GPM (1.6 to 7.1 m³/hr; 27.2 to 117.8 l/min).

  - The rotor shall complete a full 360° rotation in ~1 minute.

- High-speed, dual opposing nozzle models:

  - The rotor shall have a radius of 47' to 61' (12.2 to 18.6 m).

  - The rotor shall have a flow rate of 12.3 to 34.7 GPM (2.8 to 7.8 m³/hr; 46.5 to 131.4 l/min).

  - The rotor shall complete a full 360° rotation in ~1 minute.

3.3 Nozzles

- Conventional models shall offer 12 nozzle options.

- Opposing nozzle (ON) models shall offer 6 color-coded opposing nozzle sets.

- High-speed adjustable models shall offer 12 nozzle options.

- High-speed, dual opposing nozzle models shall offer 6 color-coded nozzles.

3.4 Additional specs

- The rotor shall have a precipitation rate of ~0.4 in/hr (15 mm/hr).

- The rotor shall have a nozzle trajectory of ~25°.

- Radius adjustment shall be performed via the stainless steel nozzle retainer screw.

- The rotor shall be serviceable without excavation.

© 2025 Hunter Industries Inc. Hunter, the Hunter logo, and other marks are trademarks of Hunter Industries Inc., registered in the U.S. and certain other countries.