# Bubblers and Bubbler Nozzles: Written Specifications

Part 1 – General

  - Hunter Bubblers and Bubbler Nozzles shall be pressure-compensating to maintain a constant output of water, regardless of the input pressure, allowing for the accurate application of water in both the ½" threaded PCB Bubblers and PCN, MSBN, and Dual-Stream Bubbler Nozzles designed for installation on Pro-Spray® pop-ups. Bubbler nozzles shall retract out of sight for a clean look when installed on sprinkler pop-ups.

- MSBN (Multi-Stream Bubbler Nozzle): This pressure-compensating nozzle delivers water through multiple streams, typically designed to gently water a planted area. The nozzle shall be threaded for use with Pro-Spray Sprinkler Bodies.

- PCN (Pressure-Compensating Nozzle): This pressure-compensating nozzle generally emits water in a more uniform "trickle" or "umbrella" pattern, delivering a steady flow that helps to water the roots without excessive runoff. The nozzle shall be threaded for use with Pro-Spray Sprinkler Bodies.

- PCB (Pressure-Compensating Bubbler): This is a fixed, above-ground, pressure-compensating water emitter. It shall feature a ½" threaded inlet for easy installation on a ½" riser.

- AFB (Adjustable Flood Bubbler): This is a fixed, above-ground, pressure-compensating water emitter. It shall feature a ½" threaded inlet for easy installation on a ½" riser. The nozzle shall offer a maximum flow rate of 2.0 GPM (7.6 l/min) that can be fine-tuned with a screw adjustment, allowing for greater flexibility.

- 5-CST-B (Dual-Stream Bubbler Nozzle): This nozzle is not pressure-compensating. It shall deliver water through 180° opposing streams and be threaded for use with Pro-Spray Sprinkler Bodies.

- Each Bubbler or Bubbler Nozzle shall have a filter screen to prevent internal system debris from entering the nozzle.

- Each Bubbler or Bubbler Nozzle shall be color-coded for easy flow identification. Colored rings for flow: Blue = 0.3 GPM (0.9 l/min), Green = 0.5 GPM (1.9 l/min), Red = 1.0 GPM (3.8 l/min), Yellow = 2.0 GPM (7.6 l/min)

Part 2 – Parts and Materials

  - Available models

Bubbler Nozzles shall be available in the following options:

- MSBN: Flow from 0.3 to 2.0 GPM (0.9 to 7.6 l/min); arc patterns from two streams to a full-circle eight streams; radius based on throw from a 4" (10 cm) spray sprinkler body.

- MSBN-25V, MSBN-50V for a two-stream V-pattern, 0.3 GPM and 0.5 GPM (0.9 and 7.6 l/min), and 1.5' to 2.5' (0.5 m to 0.8 m) radius when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- MSBN-25Q, MSBN-50Q for a three-stream quarter-circle pattern, 0.3 GPM and 0.5 GPM (0.9 and 7.6 l/min), and 1.0' to 1.5' (0.3 m to 0.5 m) radius when operating at 15 to 70 PSI (1 to 4.8 bar; 100 to 480 kPa)

- MSBN-50H, MSBN-1.0H for a five-stream half-circle pattern, 0.5 GPM and 1.0 GPM (0.9 and (3.8 l/min), and 1.0' to 1.5' (0.3 m to 0.5 m) radius when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- MSBN-10F, MSBN-20F for an eight-stream full-circle pattern, 1.0 GPM and 2.0 GPM (3.8 and 7.6 l/min), and 1.0' to 1.5' (0.3 m to 0.5 m) radius when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- PCN: Flow from 0.3 to 2.0 GPM (0.9 to 7.6 l/min); trickle and umbrella patterns

- PCN-25, trickle pattern, 0.3 GPM (0.9 l/min) when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- PCN-50, trickle pattern, 0.5 GPM (1.9 l/min) when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- PCN-10, umbrella pattern, 1.0 GPM (3.8 l/min) when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- PCN-20, umbrella pattern, 2.0 GPM (7.6 l/min) when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- PCB: Flow from 0.3 to 2.0 GPM (0.9 to 7.6 l/min); trickle and umbrella patterns, ½" thread;  available with purple reclaimed caps (PCB-xR).

- PCB-25, trickle pattern, 0.3 GPM (0.9 l/min) when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- PCB-50, trickle pattern, 0.5 GPM (1.9 l/min) when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- PCB-10, umbrella pattern, 1.0 GPM (3.8 l/min) when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- PCB-20, umbrella pattern, 2.0 GPM (7.6 l/min) when operating at 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa)

- AFB: 2.0 GPM (7.6 l/min) maximum flow; flood pattern, ½" thread

- 5-CST-B: Dual-Stream Bubbler Nozzle, 5.0' (1.5 m) radius, 0.4 GPM (1.5 l/min) when operating at 30 PSI (2.1 bar; 210 kPa)

2.2 Materials

- The bubbler shall be constructed of corrosion- and UV-resistant plastic, with an integral elastomeric flow bushing for maintaining a constant flow rate over the operating pressure range of 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa).

- Plastic material description

- The cap and body shall be made of a black acrylonitrile butadiene styrene (ABS).

- The baffle shall be made of a color-coded ABS.

- The orifice shall be made of polyvinyl chloride (PVC).

- The ABS materials shall have UV stabilizers for outdoor applications.

- Metal component materials

- The radius adjustment screw shall be made of stainless steel (applies to AFB and 5-CST-B Nozzles).

- Filter screen description

- Each Bubbler Nozzle shall come with a separate filter screen for debris protection when installed underneath the nozzle.

- The filter screen shall be made of high-density polyethylene (HDPE).

- The screen mesh (or micron) size shall be dependent on the Bubbler Nozzle model.

  - 40-mesh (420-micron) gray filter, actual opening size 0.015" (0.305 mm), Square: 
PCN, MSBN, 5-CST-B Nozzles

  - 20-mesh (840-micron) white filter, actual opening size 0.030" (0.762 mm), Square: PCB, AFB Bubbler Nozzles

- Color description

- Each MSBN or PCN Bubbler Nozzle shall be color-coded for easy flow identification. Colored rings for flow: Blue = 0.3 GPM (0.9 l/min), Green = 0.5 GPM (1.9 l/min), Red = 1.0 GPM (3.8 l/min), Yellow = 2.0 GPM (7.6 l/min).

- PCB Bubbler Nozzles and AFB have solid black bodies and caps. The PCB Nozzles can be ordered with purple caps for Reclaimed SKUS.

- The 5-CST-B Nozzle has a dark blue base and black body.

- Nozzle threads

- The PCB and AFB versions shall have a ½" Female National Pipe Thread (FNPT) inlet for connection to a ½" male-threaded riser.

- The PCN, MSBN, and 5-CST-B Nozzles shall have standard female threads that are compatible with the threaded riser on Hunter spray heads, as well as some other manufacturer’s spray heads, fit for installation in pop-up bodies having a ⅝"-28 UNS male-threaded stem at all common pop-up heights.

2.3  Warranty

1.  The nozzle shall be installed in accordance with the manufacturer’s published instructions. It shall carry a conditional 2-year exchange warranty.

Part 3 – Function and Operation

- Operating pressure

- The Bubbler Nozzles shall operate between 15 to 70 PSI (1.0 to 4.8 bar; 100 to 480 kPa).

- The 5-CST-B recommended operating pressure shall be 30 PSI (2.1 bar; 210 kPa).

- Flow rates

- Flow rates shall depend on the specific Bubbler Nozzle model.

  - Radius description

- The radius of throw shall depend on the specific Bubbler Nozzle model.

- 5-CST-B: At the recommended 30 PSI (2.1 bar; 210 kPa) operating pressure, it shall be capable of radius reduction up to 20% using a stainless steel radius adjustment screw.

- The AFB Flood Bubbler shall have a maximum flow rate of 2.0 GPM (7.6 l/min). Adjusting down shall reduce the umbrella.

- Arc adjustment

- The arcs or spray patterns shall be fixed.

© 2025 Hunter Industries Inc. Hunter, the Hunter logo, and other marks are trademarks of Hunter Industries Inc., registered in the U.S. and certain other countries.