# Commercial ROAM LR Remote Written Specifications

Part 1 – General

  - The commercial ROAM LR remote shall utilize license-free LoRa® radio technology to achieve a        2-mile (3.2 km) line-of-sight range for operation, management, and maintenance of large-scale irrigation sites. The handheld remote shall transmit a 915 MHz radio frequency to the receiver, which shall allow for connection to Hunter Industries X2™, Pro-C®, HPC, HCC, MCC, ICC2, and ACC2 Controllers via the SmartPort® Wiring Harness. The remote shall offer rugged, IP55-rated components, a user-friendly interface, intuitive controls, programmable run times from 1 to 90 minutes, and support for up to 240 stations.

Part 2 – Remote Options

  - The remote shall be available in the following options:

- ROAM LR Remote Kit with transmitter (TR), receiver (R), SmartPort Wiring Harness, four AAA batteries, and rugged carrying case

- The kit shall be Hunter Industries model ROAM-LR-KIT.

- ROAM LR Remote Transmitter (TR); transmitter sold individually

- The transmitter shall be Hunter Industries model ROAM-LR-TR.

- ROAM LR Remote Receiver (R); receiver sold individually

- The receiver shall be Hunter Industries model ROAM-LR-R.

  - Warranty

- The remote shall be installed in accordance with the manufacturer’s published instructions. The remote shall carry a conditional 3-year exchange warranty. The remote shall be the ROAM LR Remote, as manufactured for Hunter Industries Incorporated, San Marcos, California.

Part 3 – Remote Hardware

  - Transmitter

- All programming shall be completed via the handheld transmitter buttons and LCD display. This includes setting manual run times and selecting an irrigation program, station, and appropriate address for operation.

  - Receiver

- The receiver shall connect to the SmartPort Wiring Harness.

- For an outdoor-mounted controller, the receiver shall be temporarily installed on the SmartPort Wiring Harness and shall not be meant for permanent installation.

3.3 SmartPort Wiring Harness

- The SmartPort Wiring Harness shall be preinstalled in HCC, MCC, ICC2, and ACC2 Controllers. For X2, Pro-C, and HPC Controllers, the SmartPort Wiring Harness shall be connected using the red, blue, and white wires.

3.4 Kit

- The remote shall be available in kit configuration. The kit shall include the ROAM LR transmitter, receiver, SmartPort Wiring Harness, four AAA batteries, and a rugged carrying case.

Part 4 – Programming

4.0 Programming

- The remote shall have 128 programmable addresses to prevent cross-communication between multiple remotes within close proximity.

- The remote shall allow run times to be programmed from 1 to 90 minutes, which will not overwrite regular automatic programming.

- The remote shall allow manual operation of up to 240 stations.

- The remote shall achieve a 2-mile (3.2 km) line-of-sight range from transmitter to receiver.

- The remote shall transmit/operate at a 915 MHz frequency.

- The remote display shall be turned on by pressing any of the five buttons on the handheld transmitter.

- The remote display shall turn off after 30 seconds of inactivity.

4.1 User interface and display behavior

- The transmitter shall feature a large LCD display designed for visibility in outdoor environments. It shall offer intuitive, push-button controls that enable selection of stations, programs, and run time durations. A battery-life indicator shall be included on the display. The transmitter display shall automatically shut off after 30 seconds of inactivity to conserve power.

Part 5 – Technical Specifications

5.1 Frequency and range

- The system shall operate using LoRa radio technology at 915 MHz. The line-of-sight operating range shall cover up to 2 miles (3.2 km) between transmitter and receiver. Performance may vary depending on physical site conditions.

5.2 Power requirements

- The transmitter shall be powered by four AAA alkaline batteries. Battery life shall be sufficient for typical seasonal maintenance cycles.

- The receiver shall be powered by 24 VAC, 0.10 A via the controller’s power supply.

5.3 Station control

- The remote shall allow manual control of up to 240 irrigation stations. Run times may be set from 1 to 90 minutes in 1-minute increments. Programming shall not overwrite automatic schedules stored in the controller.

5.4 Compatibility

- The remote shall be compatible with Hunter Industries SmartPort-enabled controllers, including X2, Pro-C, HPC, HCC, MCC, ICC2, and ACC2 models. The remote shall not be compatible with the X-Core® Controller. The remote shall offer legacy compatibility with older controller models that include a SmartPort Wiring Harness.

5.5 Addressing and communication

- The transmitter shall support up to 128 unique programmable addresses to prevent cross-communication among remotes used on adjacent properties or work sites.

5.6 Environmental ratings

- The transmitter shall meet IP55 standards for dust and water ingress. The receiver shall be installed in accordance with manufacturer recommendations for weather protection.

  - Physical dimensions

- The transmitter shall have a height of 9.5" (241.3 mm), width of 2.3" (58.4 mm), and length of 1.0" (25.4 mm).

- The receiver shall have a height of 7.8" (198.1 mm), width of 2.0" (50.8 mm), and length of 1.0" (25.4 mm).

© 2025 Hunter Industries Inc. Hunter, the Hunter logo, and other marks are trademarks of Hunter Industries Inc., registered in the U.S. and certain other countries.