Exclusive Specification

ACCU SYNC

Fixed and adjustable regulators shall be threaded directly onto the valve

- Adjustable ACCU SYNC shall provide downstream pressure regulation from 20-100 PSI

- Fixed pressure ACCU SYNC’s shall provide either 30 PSI (2 bar; 206 kPa) or 40 PSI (2.56 bar; 276 kPa) downstream pressure

- 40 PSI (2.56 bar; 276 kPa) regulator shall be clearly marked with gray color

- 30 PSI (2 bar; 206 kPa) regulator shall be clearly marked with red color

- Adjustable ACCU SYNC shall display pressure with PSI on one side and BAR on the other

- Adjustable ACCU SYNC shall be adjustable from coined twist knob on end of body

- Adjustable ACCU SYNC shall provide indication of pressure through clear viewing window

- Regulators shall work with both Hunter AC and DC-latching solenoids

- Regulators shall work on ALL Hunter valves at ANY suggested flows