# HCC Irrigation Controller Product Specification

Part 1 – General

  - The controller shall be a full-featured commercial-industrial product with the purpose of irrigation operation, management, and monitoring of control valves and sensors. The controller shall be fully integrated with Wi-Fi connectivity to the internet and Hydrawise™ software. The controller shall be of a modular design that is provided with a base 8-station output module. The controller shall be expandable with 4-station, 8-station, or 22-station output modules up to 38 total stations (plastic enclosure) or up to 54 total stations (metal and pedestal enclosures).

Part 2 – Controller Enclosures

  - Controller shall be available in the following options:

- Plastic Wall Mount Enclosure

- The controller shall be Hunter Industries model HCC-800-PL.

- Pre-assembled controller shall have a height of 12" (30 cm), width of 14" (35 cm), and depth of 5" (12 cm).

- The controller shall be furnished in an outdoor, weather-resistant, wall mount plastic enclosure, pre-wired for remote control, with a key lock.

- The controller shall provide modular expansion from 8 to 38 stations.

- All station outputs shall have MOV and copper induction coil surge suppression.

- The enclosure shall be NEMA 3R and IP44 rated.

- A 751CH key shall be mounted in the enclosure door for security.

  - Two (2) keys shall be provided per each controller.

- Gray Powder-Coated Metal Wall Mount Enclosure/Gray Powder-Coated Metal Pedestal

- The controller shall be Hunter Industries model HCC-800-M. The metal wall mount may also be mounted on a matching gray powder-coated metal pedestal. The pedestal shall be Hunter Industries model ICC-PED.

- Pre-assembled wall mount controller shall have a height of 20" (51 cm), width of 13" (33 cm), and depth of 4.8" (12 cm).

- The controller shall be furnished in an outdoor, weather-resistant, wall mount gray powder-coated metal enclosure, pre-wired for remote control, with a key lock.

- The controller shall provide modular expansion from 8 to 54 stations.

- All station outputs shall have MOV and copper induction coil surge suppression.

- The enclosure shall be NEMA 3R and IP44 rated.

- A 751CH key shall be mounted in the enclosure door for security.

  - Two (2) keys shall be provided per each controller.

- Stainless Steel Wall Mount/Stainless Steel Pedestal

- The controller shall be Hunter Industries model HCC-800-SS. The stainless wall mount may also be mounted on a matching type 316 stainless steel pedestal. The pedestal shall be Hunter Industries model ICC-PED-SS.

- Pre-assembled wall mount controller shall have a height of 20" (51 cm), width of 13" (33 cm), and depth of 4.8" (12 cm).

- The controller shall be furnished in an outdoor, weather-resistant, type 316 stainless steel wall mount metal enclosure, pre-wired for remote control, with a key lock.

- The controller shall provide modular expansion up to 54 stations.

- The enclosure shall be NEMA 3R and IP44 rated.

- All station outputs shall have MOV and copper induction coil surge suppression.

- A 751CH key shall be mounted in the enclosure door for security.

  - Two (2) keys shall be provided per each controller.

- Plastic Pedestal

- The controller shall be Hunter Industries model HCC-800-PP.

- Pre-assembled controller shall have a height of 39" (99 cm), width of 24" (61 cm), and depth of 17" (43 cm).

- The controller shall be furnished in an outdoor plastic pedestal with removable doors, a key lock, and prewired for remote control.

- The controller shall provide modular expansion from 8 to 54 stations.

- The enclosure shall be NEMA 3R, IP34 rated, and be provided with a template and mounting hardware for installing into concrete.

- All station outputs shall have MOV and copper induction coil surge suppression.

- A 751CH key shall be mounted in the enclosure door for security.

  - Two (2) keys shall be provided per each controller.

  - Warranty

- The controller shall be installed in accordance with the manufacturer’s published instructions. The controller shall carry a conditional five-year exchange warranty. The automatic controller(s) shall be the HCC series controller as manufactured for Hunter Industries Incorporated, San Marcos, California.

Part 3 – Controller Hardware

  - Control Display

- Display shall be a 3.2" (8 cm) full graphical touch screen interface allowing for programming and manual operation.

  - Control Panel

- Operation from the control panel shall be via the touch screen only, with no available buttons or dials.

- Control panel door shall fully close and protect the wiring and internal components from moisture and dust.

  - Controller Power

- Transformer input shall be 120 VAC, 60 Hz or 230 VAC, 50 Hz, depending on requirements.

- Transformer output shall be 24 VAC, 1.4 A. The maximum output per individual station shall be 24 VAC, up to 0.56 A. Maximum output per Pump/Master Valve terminal shall be 24 VAC, up to 0.56 A.

  - Controller Surge Protection

    - The controller transformer shall be equipped with an internal, self-resetting thermal circuit breaker to protect against overheating.

    - The controller transformer shall also be equipped with a ground lug for connecting to proper earth ground hardware.

  - Station Modules

A.	Controller shall provide 4 (plastic enclosure) or 6 (metal and pedestal enclosures) separate station module slots.

- Controller shall be expandable from 8 to 38 stations (plastic) and 8 to 54 stations (metal and pedestals).

- Controller shall use 4-, 8-, or 22-station output modules.

- Station modules shall be secured against field wiring tension by locking levers.

- The controller shall have a base model capacity of 8 stations, consisting of one 8-station output module.

- Each station output shall supply 24 VAC, up to 0.56 A current for solenoid activation.

- Each station output shall have Metal Oxide Varistor (MOV) surge protection, supplemented by copper induction coils.

- The controller shall have self-diagnostic, electronic short circuit protection that detects a faulty circuit, continues watering the remainder of the schedule, and reports the faulty station via alert/notification to the user’s smartphone, tablet, or computer. The built-in milliamp sensor shall constantly be measuring the current draw of each individual station.

- Module Hardware

- The controller output modules shall have Metal Oxide Varistors (MOVs) and copper induction coils on each station output circuit to help protect the micro-circuitry from power surges.

  - Sensor Inputs

A.	The controller shall be equipped with two (2) dedicated general purpose sensor ports.

- The sensor inputs shall be compatible with any standard normally-closed or normally-open “Clik-type” sensors for automatic shutdown during rain, freeze, soil moisture, and/or wind events.

- The sensor input shall also be compatible with the Hunter HC Flow Meter for flow monitoring, alerts, and reporting.

  - Pump/Master Valve Outputs

- The controller shall have one built-in P/MV output supplied with 24 VAC, up to 0.56 A.

- The P/MV output shall be selectable as active or disabled per each individual station.

  - Common Wire

- A common wire terminal is provided on the controller’s power module, and additional commons are provided on each station output module.

  - SmartPort®

- The controller shall be pre-wired with a SmartPort® connector for easy connection of optional wireless remote controls.

- For international or short-range uses, the wireless remote control shall be the Hunter model ROAM with a useful range of up to 1,000' (305 m).

- For the United States and long-range uses where permitted, the wireless remote shall be Hunter model ROAM-XL with a useful range of up to 2 mi (3.2 km).

  - Wi-Fi Information

    - The controller shall be equipped with built-in Wi-Fi.

    - Wi-Fi operation shall be 802.11 B/G/N.

    - Wi-Fi frequency is 2.4 GHz.

    - Security shall have the ability to auto detect and offer the following security settings: WPA2, WPA Personal, and WPA Auto.

Part 4 – Programming and Operational Software

4.0 General

- The controller shall be available in an English language display. The display shall include selectable setting for date, time, and units of measurement.

- The Hydrawise software shall be fully translated and available in English, Spanish, French, Italian, German, Portuguese, Turkish, and Russian.

4.1 Programming

- The controller shall be programmed via station-based programming, up to 54 total zones available.

- Each station can have as many Start Times programmed as desired.

- The controller shall be capable of running any two stations (+ P/MV output) simultaneously.

- The controller shall have 5 weekly schedule options to choose from:

- 7-day calendar

- Up to 31-day interval calendar

- Odd day/even day programming

- Odd week/even week programming

- 365-day calendar clock to accommodate true odd-even watering

- Each station shall be programmable in minutes of Run Time, from 1 minute up to 24 hours.

- The controller shall be equipped with programmable Non-Water Days to prevent watering on selected days of the week.

- Each zone may be assigned a programmable Delay Between Stations, to allow for slow-closing valves or pressure recharging.

- Delays between stations shall be programmable in 1-second increments from 0 to 3,600 seconds (60 minutes).

- A P/MV delay shall also be programmable in 1-second increments from 0 to 60 seconds (1 minute).

4.2 Software

- The controller shall connect to Hydrawise software.

  - Hydrawise software is available via web login, and as a mobile application that is downloadable via the Apple App Store and Google Play.

- The controller shall utilize Predictive Watering™ adjustments to automatically modify irrigation scheduling based on local weather data and forecast information.

- The controller shall also have manual Seasonal Adjust settings from 0% to 300% for offline programming.