# HCC Irrigation Controller Product Specification

Part 1 – General

  - The controller shall be a full-featured commercial-industrial product for the purpose of irrigation operation, management, and monitoring of control valves and sensors. The controller shall be fully integrated with Wi-Fi connectivity to the internet and Hydrawise™ software. The controller shall be of a modular design that is provided with a standard 8-station output module. The controller shall be expandable with 4-, 8-, or 22-station conventional modules or one 54-station decoder output module.

Part 2 – Controller Hardware

  - Control Display

- Display shall be a 3.2" (8.1 cm), full graphical touchscreen interface allowing for programming and manual operation.

- All programming shall be accomplished by use of the touchscreen interface or with a smartphone, tablet, or PC.

  - Control Panel

- Operation from the control panel shall be via touchscreen only, with no buttons or dials.

  - Sensor Inputs

A.	The controller shall be equipped with two dedicated general-purpose sensor ports.

- The sensor inputs shall be compatible with any standard normally closed or normally open “Clik-type” sensors for automatic shutdown during rain, freeze, soil moisture, and/or wind events.

- The sensor inputs shall also be compatible with a designated flow sensor for flow monitoring, alerts, and reporting.

  - Pump/Master Valve Outputs

- The controller shall have one built-in P/MV (24 VAC) output with a capacity up to 0.56 A.

- The P/MV output shall be selectable as active or disabled per each individual station.

  - SmartPort®

A.	The controller shall be pre-wired with a SmartPort connector for easy connection of optional wireless remote controls.

B.	For international or short-range uses, the wireless remote control shall be the Hunter model ROAM with a useful range up to 1,000' (330 m).

C.	For uses within the United States and longer-range uses, where permitted, the wireless remote shall be Hunter model ROAM-XL with a useful range up to 2 mi. (3.2 km).

3.10 Wi-Fi Information

A.	The controller shall be equipped with built-in Wi-Fi.

B.	Wi-Fi operation shall be 802.11 b/g/n.

C.	Wi-Fi frequency is 2.4 GHz.

D.	Security shall have the ability to autodetect and offer the following security settings: WPA2, WPA Personal, and WPA Auto.

Part 4 – Programming and Operational Software

4.0 General

- The control panel shall be available in an English-language display. The display shall include selectable settings for date, time, and units of measurement.

- The Hydrawise software shall be fully translated and available in Czech, English, French, German, Greek, Hungarian, Italian, Polish, Portuguese, Russian, Spanish, Thai, and Turkish.

4.1 Programming

- The controller shall be programmed via station-based programming, up to 54 total zones available. Each zone may be named to facilitate management of large systems.

- The controller shall have 54 total start times available.

- The controller shall be capable of running any two stations (+P/MV) simultaneously.

- The controller programs shall have 5 weekly schedule options to choose from:

- 7-day calendar

- Up to 31-day interval calendar

- Odd day/even day programming

- Odd/even week programming

- 365-day calendar clock to accommodate true odd-even watering

- Each station shall be programmable in minutes of run time, from 1 minute to 24 hours.

- The controller shall be equipped with programmable Non-Water Days to prevent watering on selected days of the week.

- Each zone may be assigned a programmable Delay Between Stations, to allow for slow-closing valves or pressure recharging.

- Delays between stations shall be programmable in 1-second increments from 0 to 3,600 seconds (60 minutes).

- A P/MV delay shall be programmable in 1-second increments from 0 to 60 seconds (1 minute).

4.2 Software

- The controller shall be connected to Hydrawise software.

      - Hydrawise software is available via web login, and as a mobile application that is downloadable via the Apple® App Store℠ and Google Play™ Store.

- The controller shall utilize Predictive Watering™ adjustments to automatically modify irrigation scheduling based on local weather data and forecast information.

- The controller shall also have manual Seasonal Adjust settings from 0% to 300% for offline programming.

- The controller shall report both the monthly schedule as set by the user, and the actual irrigation performed as a result of automatic adjustments and triggers, to demonstrate water savings in response to environmental conditions.

- The controller shall provide flow sensor data to the cloud-based software, for reporting, analysis, and high- or low-flow alerts to the operator.

- The controller shall provide station-level current monitoring via a built-in milliamp sensor that shall record and display individual station electrical consumption. The software shall alert the user when out-of-range station current values are experienced for preventive maintenance.

- The controller shall feature a built-in zone tester for immediate diagnostic review of all attached stations for short circuits or overload conditions.

- The software shall allow actual photos of each station or zone location in the landscape to be attached and used for station settings.

- The software shall allow informative files to be attached to each controller record.

- The software shall allow users to create digital job sheets for service and maintenance records related to the controller.

Apple is a trademark of Apple Inc., registered in the U.S. and other countries. App Store is a service mark of Apple Inc. Google and Google Play are trademarks of Google LLC.