# HCC Irrigation Controller Written Specifications

Part 1 – General

  - The controller shall be a full-featured, commercial-industrial product for the purpose of irrigation operation, management, and monitoring of control valves and sensors. The controller shall be fully integrated with Wi-Fi connectivity to the internet and Hydrawise® software. The controller shall be of a modular design that is provided with a standard 8-station output module. The controller shall be expandable with 4-, 8-, or 22-station conventional modules or one 54-station decoder output module.

Part 2 – Controller Enclosures

  - Controller shall be available in following the options:

- Plastic wall-mount enclosure

- The controller shall be Hunter Industries model HCC-800-PL.

- Pre-assembled controller shall have a height of 12" (30.5 cm), width of 13¾" (35 cm), and a depth of 5" (12.7 cm).

- The controller shall be furnished in an outdoor, weather-resistant, wall-mount plastic enclosure, pre-wired for remote control, with a key lock.

- The controller shall provide modular expansion up to 38 stations conventionally wired, or 54 stations via two-wire decoders.

- All station outputs shall have MOV and copper induction coil surge suppression.

- The enclosure shall be IP44, NEMA 3R rated.

- A 751CH key shall be mounted in the enclosure door for security.

  - Two (2) keys shall be provided per each controller.

- Plastic pedestal-mount enclosure

- The controller shall be Hunter Industries model HCC-800-PP.

- Pre-assembled controller shall have a height of 39" (99 cm), width of 24" (61 cm), and a depth of 17" (43 cm).

- The controller shall be furnished in an outdoor, weather-resistant, pedestal-mount plastic enclosure, pre-wired for remote control, with a key lock.

- The controller shall provide modular expansion up to 54 stations conventionally wired, or 54 stations via two-wire decoders.

- All station outputs shall have MOV and copper induction coil surge suppression.

- The enclosure shall be IP34, NEMA 3R rated.

- A 751CH key shall be mounted in the enclosure door for security.

  - Two (2) keys shall be provided per each controller.

- Powder-coated metal wall-mount enclosure / Powder-coated metal pedestal

- The controller shall be Hunter Industries model HCC-800-M. The metal wall mount may also be mounted on a matching gray powder-coated metal pedestal. The pedestal shall be Hunter Industries model ICC-PED.

- Pre-assembled wall-mount controller shall have a height of 16" (40.6 cm), width of 13" (33 cm), and depth of 5" (12.7 cm).

- Pre-assembled pedestal mount shall have a height of 36" (91.4 cm), width of 11½" (29.2 cm), and depth of 5" (12.7 cm).

- The controller shall be furnished in an outdoor, weather-resistant, pedestal-mount gray metal enclosure, pre-wired for remote control, with a key lock.

- The controller shall provide modular expansion up to 54 stations conventionally wired, or 54 stations via two-wire decoders.

- All station outputs shall have MOV and copper induction coil surge suppression.

- The enclosure shall be IP44, NEMA 3R rated.

- A 751CH key shall be mounted in the enclosure door for security.

  - Two (2) keys shall be provided per each controller.

- Stainless steel wall-mount enclosure / Stainless steel pedestal

- The controller shall be Hunter Industries model HCC-800-SS. The stainless wall-mount may also be mounted on a matching type 316 stainless steel pedestal. The pedestal shall be Hunter Industries model ICC-PED-SS.

- Pre-assembled wall-mount controller shall have a height of 16" (40.6 cm), width of 13" (33 cm), and depth of 5" (12.7 cm).

- Pre-assembled pedestal mount shall have a height of 36" (91.4 cm), width of 11½" (29.2 cm), and depth of 5" (12.7 cm).

- The controller shall be furnished in an outdoor, weather-resistant, type 316 stainless pedestal-mount metal enclosure, pre-wired for remote control, with a key lock.

- The controller shall provide modular expansion up to 54 stations conventionally wired, or 54 stations via two-wire decoders.

- The enclosure shall be IP44, NEMA 3R rated.

- All station outputs shall have MOV and copper induction coil surge suppression.

- A 751CH key shall be mounted in the enclosure door for security.

  - Two (2) keys shall be provided per each controller.

  - Warranty

- The controller shall be installed in accordance with the manufacturer’s published instructions. The controller shall carry a conditional 5-year exchange warranty. The automatic controller(s) shall be the HCC series controller as manufactured for Hunter Industries Incorporated, San Marcos, California.

Part 3 – Controller Hardware

  - Control display

- Display shall be 3.2" (8.1 cm), full graphical touchscreen interface allowing for programming and manual operation.

- All programming shall be accomplished by use of the touch screen interface or with a smartphone, tablet, or PC.

  - Control panel

- Operation from the control panel shall be via touchscreen only, with no available buttons or dials.

- Control panel door shall fully close and protect the wiring and internal components from moisture and dust.

  - Controller power

- Depending on requirements, transformer input shall be 120 VAC, 60 Hz or 230 VAC, 50 Hz.

- Transformer output shall be 24 VAC, 1.4 A. Maximum output per station shall be 24 VAC, up to 0.56 A. Maximum output per P/MV terminal shall be 24 VAC, up to 0.56 A.

  - Controller surge protection

A.	The controller transformer shall be equipped with an internal, self-resetting thermal circuit breaker to protect against overheating.

B.  The controller transformer shall also be equipped with a ground lug for connecting to proper earth-ground hardware.

  - Station Modules

A.	Controller shall provide 4 (plastic enclosure) or 6 (metal and pedestal enclosures) separate station output module slots.

- Controller shall use 4-, 8-, or 22-station conventional output modules, or one 54-station decoder output module.

- Station modules shall be secured against field wiring tension by locking levers.

- Using conventional wire only, the controller shall be expandable from 8 to 38 stations (plastic) and 8 to 54 stations (metal and pedestals).

- Using conventional station modules in conjunction with one decoder output module (model: EZ-DM), all controller configurations (plastic, metal, pedestal) shall expand up to 54 stations.

- Using only one decoder output module (model: EZ-DM) all controller configurations (plastic, metal, pedestal) shall expand up to 54 stations.

- The controller shall have a base model capacity of 8 stations, consisting of one 8-station output module.

- Each station output shall supply 24 VAC, up to 0.56 A current for solenoid activation.

- The controller shall have self-diagnostic, electronic short circuit protection that detects a faulty circuit, continues watering the remainder of the program, and reports the faulty station via alert/notification to the user’s smartphone, tablet, or computer. The built-in milliamp sensor shall constantly be measuring the current draw of each individual station.

- Module hardware

- The controller output modules shall have metal oxide varistors (MOVs) and copper induction coils on each station output circuit to help protect the micro-circuitry from power surges.

  - Sensor inputs

A.	The controller shall be equipped with two dedicated general-purpose sensor ports.

- The sensor inputs shall be compatible with any standard normally closed or normally open “Clik-type” sensors for automatic shutdown during rain, freeze, soil moisture, and/or wind events.

- The sensor inputs shall also be compatible with the Hunter HC Flow Meter for flow monitoring, alerts, and reporting.

  - P/MV outputs

- The controller shall have one built-in P/MV (24 VAC) output with a capacity of up to 0.56 A.

- The P/MV output shall be selectable as active or disabled per each individual station.

  - Common wire

- A common wire terminal is provided on the controller’s power module, and additional commons are provided on each station output module.

  - SmartPort®

A.	The controller shall be pre-wired with a SmartPort connector for easy connection of optional wireless remote controls.

B.	For international or short-range uses, the wireless remote control shall be the Hunter model ROAM with a useful range of up to 1,000' (330 m).

C.	For use in the United States or long-range uses, where permitted, the wireless remote shall be Hunter model ROAM-XL with a useful range of up to 2 mi. (3.2 km).

3.10  Wi-Fi information

A.	The controller shall be equipped with built-in Wi-Fi.

B.	Wi-Fi operation shall be 802.11 b/g/n.

C.	Wi-Fi frequency is 2.4 GHz.

D.	Security shall have the ability to auto detect and offer the following security settings: WPA2, WPA Personal, and WPA Auto.

Part 4 – Programming and Operational Software

4.1 General

- The control panel shall be available in an English-language display. The display shall include selectable settings for date, time, and units of measurement.

- The Hydrawise software shall be fully translated and available in Czech, English, French, German, Greek, Hungarian, Italian, Polish, Portuguese, Russian, Spanish, Thai, and Turkish.

4.2 Programming

- The controller shall be programmed via station-based programming, up to 54 total zones available.

- The controller shall have 54 total Start Times available.

- The controller shall be capable of running any two stations (+P/MV) simultaneously.

- The controller programs shall have 5 weekly schedule options to choose from:

- 7-day calendar

- Up to 31-day interval calendar

- Odd-day/even-day programming

- Odd-week/even-week programming

- 365-day calendar clock to accommodate true odd-even watering

- Each station shall be programmable in minutes of run time, from 1 minute to 24 hours.

- The controller shall be equipped with programmable Non-Water Days to prevent watering on selected days of the week.

- Each zone may be assigned a programmable Delay Between Stations, to allow for slow-closing valves or pressure recharging.

- Delays between stations shall be programmable in 1-second increments from 0 to 3,600 seconds (60 minutes).

- A P/MV delay shall be programmable in 1-second increments from 0 to 60 seconds (1 minute).

4.3 Software

- The controller shall be connected to Hydrawise software.

      - Hydrawise software is available via web login, and as a mobile application that is downloadable via the Apple® App Store and Google Play™ Store.

- The controller shall utilize Predictive Watering® adjustments to automatically modify irrigation scheduling based on local weather data and forecast information.

- The controller shall also have manual Seasonal Adjust settings from 0% to 300% for offline programming.

Apple is a trademark of Apple Inc., registered in the U.S. and other countries. App Store is a service mark of Apple Inc. Google and Google Play are trademarks of Google LLC.

© 2020 Hunter Industries Inc. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.