# Pro-HC Irrigation Controller Written Specifications

Part 1 – General

  - The controller shall be a full-featured residential/light commercial product for the purpose of irrigation operation, management, and monitoring of control valves and sensors. The controller shall be fully integrated with Wi-Fi connectivity to the internet and Hydrawise® software. The controller shall be of a fixed-station design that is available in 6-, 12-, and 24-station models.

Part 2 – Controller Enclosures

  - Controller shall be available in following the options:

- Plastic wall-mount indoor enclosure

- The controller shall be Hunter Industries model PHC-600-i, PHC-1200-i, or                   PHC-2400-i.

- Pre-assembled controller shall have a height of 8¼" (21 cm), width of 9½" (24 cm), and a depth of 3½" (8.8 cm).

- The controller shall be furnished in a wall-mount plastic enclosure suitable for remote control with a smartphone.

- The controller shall be a fixed-station controller with 6, 12, or 24 stations.

- All station outputs shall have MOV and copper induction coil surge suppression.

- Plastic wall-mount outdoor enclosure

- The controller shall be Hunter Industries model PHC-600, PHC-1200, or PHC-2400.

- Pre-assembled controller shall have a height of 9" (22.8 cm), width of 10" (25 cm), and depth of 4" (10 cm).

- The controller shall be furnished in an outdoor, weather-resistant, wall-mount plastic enclosure, suitable for remote control with a smartphone, with a key lock.

- The controller shall be a fixed-station controller with 6, 12, or 24 stations.

- All station outputs shall have MOV and copper induction coil surge suppression.

- A 751CH key shall be mounted in the enclosure door for security.

  - Two (2) keys shall be provided per each controller.

  - Warranty

- The controller shall be installed in accordance with the manufacturer’s published instructions. The controller shall carry a conditional 2-year exchange warranty. The automatic controller(s) shall be the Pro-HC series controller as manufactured for Hunter Industries Incorporated, San Marcos, California.

Part 3 – Controller Hardware

  - Control display

- Display shall be 2.9" (7.4 cm), full-color graphical touchscreen interface allowing for programming and manual operation.

- All programming shall be accomplished by use of the touchscreen or with a smartphone, tablet, or PC.

  - Control panel

- Operation from the control panel shall be via touchscreen only, with no available buttons or dials.

- The control panel door shall fully close and protect the wiring and internal components from moisture and dust.

  - Controller power

- Depending on requirements, transformer input shall be 120 VAC, 60 Hz or 230 VAC, 50 Hz.

- Transformer output shall be 24 VAC, 1A. Maximum output per station shall be 24 VAC, up to 0.56 A. Maximum output per P/MV terminal shall be 24 VAC, up to 0.28 A.

  - Controller surge protection

A.	The controller transformer shall be equipped with an internal, self-resetting thermal circuit breaker to protect against overheating.

  - Sensor inputs

A.	The controller shall be equipped with two dedicated general-purpose sensor ports.

- The sensor inputs shall be compatible with any standard normally closed or normally-open “Clik-type” sensors for automatic shutdown during rain, freeze, soil moisture, and/or wind events.

- The sensor inputs shall also be compatible with the Hunter HC Flow Meter for flow monitoring, alerts, and reporting.

  - P/MV outputs

- The controller shall have one built-in P/MV (24 VAC) output with a capacity of up to 0.28 A.

  - Common wire

- For both 6- and 12-station controllers, one fixed common wire terminal is provided.

- For the 24-station controller, two fixed common wire terminals are provided.

Part 4 – Programming and Operational Software

4.1	General

- The control panel shall be available in an English-language display. The display shall include selectable settings for date, time, and units of measurement.

- The Hydrawise software shall be fully translated and available in Czech, English, French, German, Greek, Hungarian, Italian, Polish, Portuguese, Russian, Spanish, Thai, and Turkish.

4.2	Programming

- The controller shall be programmed via station-based programming, up to 6, 12, or 24 total zones available.

- The controller shall have 36 total Start Times available.

- The controller shall be capable of running any one station (+P/MV) at a time.

- The controller programs shall have 5 weekly schedule options to choose from:

- 7-day calendar

- Up to 31-day interval calendar

- Odd-day/even-day programming

- Odd-week/even-week programming

- 365-day calendar clock to accommodate true odd-even watering

- Each station shall be programmable in minutes of run time, from 1 minute to 24 hours.

- The controller shall be equipped with programmable Non-Water Days to prevent watering on selected days of the week.

- Each zone may be assigned a programmable Delay Between Stations, to allow for slow-closing valves or pressure recharging.

- Delays between stations shall be programmable in 1-second increments from 0 to 3,600 seconds (60 minutes).

- A P/MV delay shall be programmable in 1-second increments from 0 to 60 seconds (1 minute).

4.3 Software

- The controller shall be connected to Hydrawise software.

      - Hydrawise software is available via web login, and as a mobile application that is downloadable via the Apple® App Store and Google Play™ Store.

- The controller shall utilize Predictive Watering® adjustments to automatically modify irrigation scheduling based on local weather data and forecast information.

- The controller shall also have manual Seasonal Adjust settings from 0% to 300% for offline programming.

Apple is a trademark of Apple Inc., registered in the U.S. and other countries. App Store is a service mark of Apple Inc. Google and Google Play are trademarks of Google LLC.

© 2020 Hunter Industries Inc. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.