# I-40 Written Specifications

Part 1 – General

1.1  The I-40 is a rotary gear-driven sprinkler for commercial applications. It features three port nozzle options, a non-strippable drive, a thick rubber cover to prevent injury, easy arc adjustment, and optional reclaimed water ID.

Part 2 – Parts and Material

2.1  The I-40 shall be available in the following versions: 4" (10 cm) stainless steel riser, 6" (15 cm) stainless steel riser.

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall be equipped with a durable, stainless-steel riser spring.

- The sprinkler shall be available with an optional opposing nozzle (ON) housing.

- The sprinkler shall be available with 12 nozzle choices in flow rates from 7.6 to 29.5 GPM (1.63 to 6.84 m3/hr; 27.2 to 114.1 l/min) for conventional models and 13.0 to 33.7 GPM (2.75 to 7.76 m3/hr; 45.8 to 129.4 l/min) for ON models.

- The sprinkler shall be equipped with a non-strippable gear drive.

- The sprinkler shall be equipped with automatic arc return.

- The arc of the sprinkler shall be adjustable from 50° to 360°.

- The sprinkler shall be equipped with through-the-top arc adjustment.

- The sprinkler shall be equipped with a QuickCheck™ arc mechanism.

- The sprinkler shall be equipped with a factory-installed rubber cover.

- The sprinkler shall be equipped with a standard drain check valve to hold back up to 15' 
(4.5 m) of elevation.

- The sprinkler shall be available in a reclaimed water ID version.

- The sprinkler shall be available in a high-speed version.

2.2.  Rotor dimensions

A. I-40-04

    - Pop-up height: 4" (10 cm)

    - Overall height: 7⅞" (20 cm)

    - Exposed diameter: 2" (5 cm)

    - Inlet size: 1" (25 mm) NPT

B. I-40-06

    - Pop-up height: 6" (15 cm)

    - Overall height: 10¼" (26 cm)

    - Exposed diameter: 2" (5 cm)

    - Inlet size: 1" (25 mm) NPT

2.3  Warranty

A.  The sprinkler shall carry a five-year exchange warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 40 to 100 PSI (2.5 to 7.0 bar; 250 to 700 kPa)

A.  Operating pressure range: 40 to 100 PSI (2.5 to 7.0 bar; 250 to 700 kPa)

3.2  Radius: 44' to 69' (13.1 to 21.3 m) conventional, 52' to 76' (15.2 to 23.2 m) ON

3.3  Flow rate: 7.6 to 29.5 GPM (1.63 to 6.84 m3/hr; 27.2 to 114.1 l/min) conventional, 13.0 to 33.7 GPM (2.75 to 7.76 m3/hr; 45.8 to 129.4 l/min) ON

3.4  Precipitation rate: 0.4 in/hr (15 mm/hr) approximately

3.5  Nozzle trajectory: 25° approximately

© 2019 Hunter Industries Inc. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.