# MP Rotator® Nozzle Written Specifications

Part 1 – General

  - The MP Rotator is a high-efficiency, multi-stream, multi-trajectory rotary nozzle designed with a viscous drive for rotation. The multiple streams allow for a slow application of water without misting at a high distribution uniformity. Standard MP Rotator nozzles, designated by a black canister, have a matched precipitation rate of approximately 0.4 in/hr (10 mm/hr) across any arc and radius to better match soil intake rates and prevent runoff. MP800 nozzles, designated by a gray canister, have a matched precipitation rate of approximately 0.8 in/hr (20 mm/hr). MP Rotator nozzles have female threads for installation on male-threaded pop-up sprinklers. Select models have a male-threaded option for installation on female-threaded pop-up sprinklers. Each MP Rotator nozzle shall have a filter screen to prevent internal system debris from entering the nozzle and the patented double-pop feature to prevent external debris from falling into the nozzle. Each MP Rotator nozzle shall be color-coded for easy field identification.

Part 2 – Parts and Material

- MP Rotator nozzles shall be available in the following options:

- Standard MP Rotator nozzles: approximately 0.4 in/hr (10 mm/hr) precipitation rate

- MP-1000-90, MP-1000-210, MP-1000-360 for an 8–15' (2.5–4.5 m) radius when operating at 30–55 PSI (2.1–3.8 bar; 210–380 kPa)

- MP-2000-90, MP-2000-210, MP-2000-360 for a 13–21' (4.0–6.4 m) radius when operating at 25–55 PSI (1.7–3.8 bar; 170–380 kPa)

- MP-3000-90, MP-3000-210, MP-3000-360 for a 22–30' (6.7–9.1 m) radius when operating at 25–55 PSI (1.7–3.8 bar; 170–380 kPa)

- MP-3500-90 for a 31–35' (9.4–10.7 m) radius when operating at 25–55 PSI (1.7–3.8 bar; 170–380 kPa)

- MP-CORNER for an 8-15' (2.5–4.5 m) radius when operating at 25–55 PSI (1.7–3.8 bar; 170–380 kPa)

- MP Rotator Strip nozzles: precipitation rate dependent on layout

- MP-LCS-515, MP-RCS-515, MP-SS-530 for 5' (1.5 m) wide strip models

- MP800 nozzles: approximately 0.8 in/hr (20 mm/hr) precipitation rate

- MP-800SR-90, MP-800SR-360 for 6–12' (1.8–3.5 m) radius when operating at 30–55 PSI (2.1–3.8 bar; 210–380 kPa)

- MP-815-90, MP-815-210, MP-815-360 for 8–15' (2.5–4.9 m) radius when operating at 30–55 PSI (2.1–3.8 bar; 210–380 kPa)

- Plastic material description

- The adjustable orifice shall be manufactured from polyurethane and acetal plastic materials for durability and adjustability.

- The acetal material shall have UV stabilizers for outdoor applications.

- Metal component materials

- The radius adjustment screw, arc ring, spring, and internal collar shall be made of stainless steel.

- The stator that drives the speed of rotation inside the silicone chamber shall be made of brass.

- Filter screen description

- Each MP Rotator shall come with a detachable filter screen.

- The filter screens shall be made of polypropylene.

- The screen mesh size shall be dependent on the MP Rotator model.

  - 60 mesh: MP800SR90

  - 40 mesh: MP1000, MP2000, MP Corner, MP Strips, MP800SR360, MP815

  - 20 mesh: MP3000, MP3500

- Color description

  - Each MP Rotator model shall have its own designated color scheme.

  - Standard MP Rotator nozzles have a black canister and black top retainer.

- MP-1000-90 (maroon), MP-1000-210 (light blue), MP-1000-360 (olive)

- MP-2000-90 (black), MP-2000-210 (green), MP-2000-360 (red)

- MP-3000-90 (blue), MP-3000-210 (yellow), MP-3000-360 (gray)

- MP-3500-90 (tan)

- MP-Corner (turquoise)

- MP-LCS-515 (ivory), MP-RCS-515 (copper), MP-SS-530 (brown)

  - The MP800 family has a gray canister and gray top retainer.

- MP-800SR-90 (orange), MP-800SR-360 (lime green)

- MP-815-90 (maroon), MP-815-210 (light blue), MP-815-360 (olive)

- Nozzle threads

- Models MP1000, MP2000, MP3000, MP3500, MP Corner, MP Strips, MP800SR, and MP815 shall be fit for installation in pop-up bodies having a 5/8-27 UNS male-threaded stem at all common pop-up heights.

- Models MP1000HT, MP2000HT, MP3000HT, MP Corner HT, and MP Strip HT shall be fit for installation in pop-up bodies having a 5/8-28 UNS female-threaded stem at all common pop-up heights.

- Viscous drive

- The viscous fluid used to maintain the rotation speed of the MP Rotator shall be made of a silicone material.

- The silicone chamber shall be sealed with EPDM rubber seals.

- The brass stator inside the silicone chamber shall control the rotation speed.

  - Warranty

- MP Rotator nozzles shall have a warranty period of three years.

Part 3 – Function and Operation

- Operating pressure

- MP Rotator nozzles shall operate between 25–55 PSI (1.7–3.8 bar; 170–380 kPa).

- The recommended operating pressure is 40 PSI (2.8 bar; 280 kPa).

- Flow rates

- Flow rates shall depend on the specific MP Rotator model.

- As the arc and radius are adjusted, the flow rate shall change to maintain matched precipitation.

  - Radius description

- The radius of throw shall depend on the specific MP Rotator model.

- At the recommended 40 PSI (2.8 bar; 280 kPa) operating pressure, full- or part-circle sprinklers shall be capable of radius reduction up to 25% using a stainless steel radius adjustment screw.

- The radius reduction screw shall have a slip clutch mechanism to prevent internal damage if turned past the minimum or maximum radius settings.

- The radius reduction screw shall reduce the pressure and flow upstream of the adjustable orifice thereby maintaining stream integrity.

- Arc adjustment

- Depending on the model selected, the part-circle sprinkler shall have an infinitely adjustable arc from 45° to 105°, 90° to 210°, or 210° to 270° using the stainless steel arc ring.

- The full-circle sprinkler shall irrigate a full 360°.

- The 45° to 105° model shall not require coverage from adjacent sprinklers closer than 3' 
(1 m) from the head.

- Arc adjustment shall be effective only while the sprinkler is popped up and shall be ineffective when the sprinkler is popped down.

- The adjustment mechanism shall have a ratcheting action to prevent internal damage when turned past the minimum or maximum arc limits.

  - Application rate

    - Models MP1000, MP2000, MP3000, MP3500, MP Corner, and MP Strips shall produce and maintain a matched precipitation rate no greater than 0.6 in/hr (15 mm/hr) throughout the arc adjustment range and radius adjustment range, with a radius reduction up to 25%, when spaced at 50% of irrigated diameter.

    - Models MP800SR and MP815 shall produce and maintain a matched precipitation rate no greater than 1.0 in/hr (25 mm/hr) throughout the arc adjustment range and radius adjustment range, with a radius reduction up to 25%, when spaced at 50% of irrigated diameter.

  - Double-pop feature

    - When installed in a pop-up sprinkler body, the MP Rotator rotor shall pop up after the body stem is fully extended; upon decreasing pressure, the MP Rotator rotor shall retract before the retraction of the sprinkler body stem.

    - MP Rotator nozzles shall pop up at approximately 15 PSI (1.0 bar; 100 kPa).

© 2019 Hunter Industries Inc. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.