# PGJ Written Specifications

Part 1 – General

1.1  The PGJ is a compact, rotary, gear-driven sprinkler with a footprint similar in size to a spray head. It features water-efficient nozzles that can be easily changed, simple arc adjustment, and a rubber cover.

Part 2 – Parts and Material

2.1  The PGJ shall be available in the following versions: Shrub, 4" (10 cm) pop-up, 6" (15 cm) pop-up and 12" (30 cm) pop-up.

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall be equipped with a durable, stainless steel riser spring.

- The sprinkler shall be available with 8 nozzle choices in flow rates from 0.75 to 5.0 GPM (0.13 to 1.23 m3/hr; 2.2 to 20.5 l/min).

- The arc of the sprinkler shall be adjustable from 40° to 360°.

- The sprinkler shall be equipped with through-the-top arc adjustment.

- The sprinkler shall be equipped with a QuickCheck™ arc mechanism.

- The sprinkler shall be equipped with a factory-installed rubber cover.

- The sprinkler may be equipped with an optional drain check valve to hold back up to 7' 
(2.1 m) of elevation.

- The sprinkler shall be available in a reclaimed water ID version.

2.2.  Rotor dimensions

A. PGJ-00

    - Overall height: 7" (18 cm)

    - Exposed diameter: 1⅛" (3 cm)

    - Inlet size: ½" NPT

B. PGJ-04

    - Pop-up height: 4" (10 cm)

    - Overall height: 7⅛" (18 cm)

    - Exposed diameter: 1⅛" (3 cm)

    - Inlet size: ½" NPT

C. PGJ-06

    - Pop-up height: 6" (15 cm)

    - Overall height: 9⅛" (23 cm)

    - Exposed diameter: 1⅛" (3 cm)

    - Inlet size: ½" NPT

D. PGJ-12

    - Pop-up height: 12" (30 cm)

    - Overall height: 16⅜" (41 cm)

    - Exposed diameter: 1⅛" (3 cm)

    - Inlet size: ½" NPT

2.3  Warranty

A.  The sprinkler shall carry a two-year exchange warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 30 to 50 PSI	(1.7 to 3.8 bar; 170 to 380 kPa)

A.  Operating pressure range: 20 to 100 PSI (1.4 to 7.0 bar; 140 to 700 kPa)

3.2  Radius: 15' to 37' (4.3 to 11.6 m)

3.3  Flow rate: 0.64 to 5.3 GPM (0.13 to 1.23 m3/hr; 2.2 to 20.5 l/min)

3.4  Precipitation rate: 0.6 in/hr (15 mm/hr) approximately

3.5  Nozzle trajectory: 14° approximately

© 2019 Hunter Industries Inc. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.