# PGP® Written Specifications

Part 1 – General

1.1  The PGP is a rotary gear-driven sprinkler for residential and light commercial applications. It features water-efficient nozzles that can be easily changed and simple arc adjustment.

Part 2 – Parts and Material

2.1  The PGP shall be available in the following versions: 4" (10 cm) pop-up.

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall be equipped with a durable stainless-steel riser spring.

- The sprinkler shall be available with 27 nozzle choices in flow rates from 0.5 to 14.1 GPM (0.10 to 3.22 m3/hr; 1.7 to 53.7 l/min).

- The arc of the sprinkler shall be adjustable from 40° to 360°.

- The sprinkler shall be equipped with through-the-top arc adjustment.

- The sprinkler shall be equipped with a QuickCheck™ arc mechanism.

- The sprinkler shall be equipped with a factory-installed rubber cover.

2.2.  Rotor dimensions

A. PGP-ADJ

    - Pop-up height: 4" (10 cm)

    - Overall height: 7⅜" (19 cm)

    - Exposed diameter: 1¾" (4 cm)

    - Inlet size: ¾" NPT

2.3  Warranty

A.  The sprinkler shall carry a two-year exchange warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 25 to 70 PSI	(1.7 to 4.5 bar; 170 to 450 kPa)

A.  Operating pressure range: 20 to 100 PSI (1.4 to 1.7 bar; 140 to 700 kPa)

3.2  Radius: 22' to 52' (6.4 to 15.8 m)

3.3  Flow rate: 0.5 to 14.1 GPM (0.10 to 3.22 m3/hr; 1.7 to 53.7 l/min)

3.4  Precipitation rate: 0.4 in/hr (10 mm/hr) approximately

3.5  Nozzle trajectory: Standard = 25° approximately, Low-angle = 13° approximately

© 2019 Hunter Industries Inc. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.