# PGP Ultra Written Specification

Part 1 – General

1.1  The PGP Ultra is a rotary gear driven sprinkler for residential and light commercial applications.  It features a non-strippable drive, automatic arc return, easily changed, water efficient nozzles and easy arc adjustment.

Part 2 – Parts and Material

2.1  The PGP Ultra Shall be available in the following versions: Shrub, 4” (10 cm) Pop-Up, 6” (15 cm) Pop-Up, 12” (30 cm) Pop-up.

- The sprinkler shall be constructed from corrosion and impact resistant ABS plastic.

- The sprinkler shall be equipped with a durable stainless-steel riser spring.

- The sprinkler shall be available with 34 nozzle choices in flow rates from 0.36 to 14.8 GPM (0.07 to 3.23 m3/hr; 1.2 to 53.8 l/min).

- The sprinkler shall be equipped with a non-strippable gear drive.

- The sprinkler shall be equipped with automatic arc return.

- The arc of the sprinkler shall be adjustable from 50-360 degrees

- The sprinkler shall be equipped with through the top arc adjustment.

- The sprinkler shall be equipped with a quick check arc mechanism.

- The sprinkler shall be equipped with a factory installed rubber cover.

- The sprinkler may be equipped with an optional drain check valve to hold back up to 10’ (2.1 m) of elevation.

- The sprinkler shall be available in a reclaimed water ID version.

2.2.  Rotor dimensions

PGP-00

Overall height: 7 ½” (19 cm)

Exposed diameter: 1-3/4” (4.5 cm)

Inlet size: 3/4” NPT

PGP-04

Pop-up height: 4” (10 cm)

Overall height: 7-1/2” (19 cm)

Exposed diameter: 1-3/4” (4.5 cm)

Inlet size: 3/4” NPT

PGP-06

Pop-up height: 6” (15 cm)

Overall height: 9-7/8” (25 cm)

Exposed diameter: 1-3/4” (4.5 cm)

Inlet size: 3/4” NPT

PGP-12

Pop-up height: 12” (30 cm)

Overall height: 17” (43 cm)

Exposed diameter: 1-3/4” (4.5 cm)

Inlet size: 3/4” NPT

2.3  Warranty

A.  The sprinkler shall carry a five-year, exchange warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 25 to 70 PSI	(1.7 to 4.5 bar; 170 to 450 kPa)

A.  Operating pressure range: 20 to 100 PSI (1.4 to 7.0 bar; 140 to 700 kPa)

3.2  Radius: 17’ to 47’ (4.9 to 14.0 m)

3.3  Flow rate: 0.36 to 14.8 GPM (0.07 to 3.23 m3/hr; 1.2 to 53.8 l/min)

3.4  Precipitation rate: 0.4 in/hr approx. (10 mm/hr approx.)

3.5  Nozzle trajectory: Std = 25 degrees approx., Low angle = 13 degrees approx..