# SRM Written Specification

Part 1 – General

1.1  The SRM is a compact, short range, rotary gear driven sprinkler.  It features easily changed, water efficient nozzles and easy arc adjustment.

Part 2 – Parts and Material

2.1  The SRM Shall be available in the following versions: 4” Pop-Up (10 cm).

- The sprinkler shall be constructed from corrosion and impact resistant ABS plastic.

- The sprinkler shall be equipped with a durable stainless-steel riser spring.

- The sprinkler shall be available with 8 nozzle choices in flow rates from 0.42 to 4.3 GPM (0.08 to 1.0 m3/hr; 1.4 to 16.7 l/min).

- The arc of the sprinkler shall be adjustable from 40-360 degrees

- The sprinkler shall be equipped with through the top arc adjustment.

- The sprinkler shall be equipped with a quick check arc mechanism.

2.2.  Rotor dimensions

SRM-04

Pop-up height: 4” (10 cm)

Overall height: 7-1/8” (18 cm)

Exposed diameter: 1-1/8” (3 cm)

Inlet size: ½” NPT

2.3  Warranty

A.  The sprinkler shall carry a one-year, exchange warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 30 to 50 PSI	(1.7 to 3.8 bar; 170 to 380 kPa)

A.  Operating pressure range: 20 to 100 PSI (1.4 to 7.0 bar; 140 to 700 kPa)

3.2  Radius: 14’ to 34’ (4.3 to 10.7 m)

3.3  Flow rate: 0.42 to 4.3 GPM (0.08 to 1.0 m3/hr; 1.4 to 16.7 l/min)

3.4  Precipitation rate: 0.6 in/hr approx. (15 mm/hr)

3.5  Nozzle trajectory: 15 degrees approx.