Exclusive Specification

Micro Irrigation

PCZ-101

The kit shall be pre-assembled with stainless steel filtration and pressure regulation.

- The kit shall feature a water tested 1” x ¾” inline valve.

- The kit shall feature a Senninger pressure regulator.

- The kit shall be available in 25 PSI or 40 PSI (1.7 or 2.8 bar; 170 or 280 kPa) regulator versions

- The kit shall have a flow of 0.5 to 15 GPM (30 to 900 GPH) (2 to 55 l/min).

- The kit shall have an operating pressure of 20 to 120 PSI (1.4 to 8.0 bar; 140 to 800 kPa).

- The kit shall have an operating temperature up to 120 F (66 C).

- The kit shall feature a 1” x ¾” filter-regulator.

- The kit shall feature a 150 mesh (100 microns) stainless steel screen.

ACZ

The kit shall be pre-assembled with filtration and pressure regulation for above ground installation.

ACZ-075

- The kit shall feature a ¾” x ¾” anti-siphon valve

- The kit shall be available in 25 PSI or 40 PSI (1.7 or 2.8 bar; 170 or 280 kPa) regulator versions.

- The kit shall have a flow of 0.5 to 15 GPM (30 to 900 GPH) (2 to 55 l/min).

- The kit shall have an operating pressure of 20 to 120 PSI (1.4 to 8.0 bar; 140 to 800 kPa).

- The kit shall have an operating temperature up to 120 F (66 C).

- The kit shall feature a 150 mesh (100 microns) stainless steel screen.

ACZ-101

- The kit shall feature a 1” anti-siphon valve

- The kit shall be available in 25 PSI or 40 PSI (1.7 or 2.8 bar; 170 or 280 kPa) regulator versions.

- The kit shall have a flow of 0.5 to 15 GPM (30 to 900 GPH) (2 to 55 l/min).

- The kit shall have an operating pressure of 20 to 120 PSI (1.4 to 8.0 bar; 140 to 800 kPa).

- The kit shall have an operating temperature up to 120 F (66 C).

- The kit shall feature a 1” x ¾” filter-regulator.

- The kit shall feature a 150 mesh (100 microns) stainless steel screen.

1” ICZ

The kit shall be offered in durable low, medium and high-flow versions.

ICZ-101

- The kit shall offer factory-installed Filter Sentry.

- The kit shall be available in 25 PSI or 40 PSI (1.7 or 2.8 bar; 170 or 280 kPa) regulator versions.

- The kit shall have a flow of 2 to 20 GPM (120 to 1,200 GPH)

- The kit shall have an operating pressure upto 120 PSI (8.0 bar; 800 kPa).

- The kit shall have an operating temperature of up to 120 F (66 C).

- The kit shall feature a 150 mesh (100 microns) stainless steel screen.

ICZ-101-LF

- The kit shall be available in 25 PSI or 40 PSI (1.7 or 2.8 bar; 170 or 280 kPa) regulator versions.

- The kit shall have a flow of 0.5 to 15 GPM (30 to 900 GPH)

- The kit shall have an operating pressure upto 120 PSI (8.0 bar; 800 kPa).

- The kit shall have an operating temperature of up to 120 F (66 C).

- The kit shall feature a 150 mesh (100 microns) stainless steel screen.

- The kit shall feature a 1” inlet and ¾” outlet. (You could use this single statement for the PCZ kits and the 1” ACZ kits if you wanted.  Since you used the valve sizes above, I added the filter size.  However, this single statement would consolidate if it made sense.)

ICZ-101-LF-R

- The kit shall be available in 25 PSI or 40 PSI (1.7 or 2.8 bar; 170 or 280 kPa) regulator versions.

- The kit shall have a flow of 0.5 to 15 GPM (30 to 900 GPH)

- The kit shall have an operating pressure upto 120 PSI (8.0 bar; 800 kPa).

- The kit shall have an operating temperature of up to 120 F (66 C).

- The kit shall feature a 150 mesh (100 microns) stainless steel screen.

- The kit shall feature a 1” inlet and ¾” outlet.

- The kit shall feature a reclaimed water ID tag, purple flow control knob, Filter Sentry, and purple chlorine-resistant diaphragm.

ICZ-151

The kit shall be constructed of glass-filled Nylon with a fabric-reinforced diaphragm, and a double-beaded diaphragm seal.

ICZ-151-40-XL

- The kit shall be factory assembled.

- The kit shall be pressure regulated to 40 PSI.

- The kit shall have a flow of 20 to 80 GPM.

- The kit shall have an operating pressure of up to 120 PSI.

- The kit shall have an operating temperature of up to 120 F.

- The kit shall feature a 150 mesh (100 microns) stainless steel screen.

- Optional 120mesh (125 micron) disc filter is available

- The kit shall feature a 1 1/2” inlet and 2” outlet.

ICZ-201

The kit shall be constructed of glass-filled Nylon with a fabric-reinforced diaphragm, and a double-beaded diaphragm seal.

ICZ-201-40-XL

- The kit shall be factory assembled.

- The kit shall be pressure regulated to 40 PSI.

- The kit shall have a flow of 20 to 100 GPM.

- The kit shall have an operating pressure of up to 120 PSI.

- The kit shall have an operating temperature of up to 120 F.

- The kit shall feature a 150 mesh (100 microns) stainless steel screen.

- Optional 120mesh (125 micron) disc filter is available

FILTER REGULATORS

The filter/regulator combo shall feature a Senninger regulator and a stainless-steel filter screen.

HFR

- No assembly other than installing the bonnet. The filter shall be available in 25 PSI or 40 PSI (1.7 or 2.8 bar; 170 or 280 kPa) regulator versions.

- The filter shall be available in 1” x ¾” and ¾” x ¾” versions.

- The kit shall feature a 150 mesh (100 microns) stainless steel screen.

HFR-R

- The filter shall be available in 25 PSI or 40 PSI regulator versions

- The filter shall be available in 1” x ¾” and ¾” x ¾” versions.

- The kit shall feature a 150 mesh SS screen

- The kit shall feature a purple bonnet without a flush cap.

FILTERS – ¾” & 1”

The filter assembly shall feature a polypropylene body and stainless-steel filter screen.

HY-075, HY-100, HY-100-075, HY-100-R

- The filter shall feature a 150 mesh (100 microns) stainless steel screen.

- The filter shall have an operating pressure up to 120 PSI.

- The filter shall also be offered with a reclaimed purple bonnet without flush cap.

FILTERS – 1 ½” & 2”

The filter assembly shall feature a glass-filled polypropylene body and stainless-steel filter screen.

HY-151, HY-151-D, HY-201, HY-201-D

- The filter assembly shall feature a 150 mesh (100 microns) stainless steel screen.

- The filter assembly shall have an operating pressure up to 150 PSI.

- The filer assembly shall have optional screens of 80 and 120 mesh.

- The filter assembly shall have an available disk filter (125 microns).

SENNINER PRESSURE REGULATORS

The pressure regulators maintain consistent preset outlet pressure to prevent damage to system components.

- The pressure regulator shall be 100% water tested.

- The pressure regulator shall be capable of above or below ground installation.

- The pressure regulator shall feature tamper-proof construction.

HDL-CV

The drip line shall feature pressure compensation with check valve, flow indication stripes, and a 6’ check height.

- The drip line shall feature an anti-siphon feature.

- The drip line shall feature 3 color-coded stripes to indicate flow.

- The drip line shall be UV resistant.

- The CV dripline shall be distinguished from other HDL by its dark brown base resin.

- The drip line shall be offered in stretch-wrapped coils.

- The drip line shall utilize flat emitters with multiple inlet filters.

- The dripline shall feature a check valve which holds the water to a height of at least 6 feet.

HDL-PC & HDL-R

The drip line shall feature construction of robust materials and pressure compensation.

- The drip line shall feature color-coded stripes to indicate flow.

- The drip line shall be UV resistant.

- The PC dripline shall be distinguished from other HDL by its light brown base resin.

- The drip line shall be offered in stretch-wrapped coils.

- The drip line shall utilize flat emitters with multiple inlet filters.

- The drip line shall be available in a reclaimed version identified by purple stripes to indicate irrigation using non-potable water.

HDL-COP

The drip line shall feature copper oxide to minimize root intrusion.

- The drip line shall feature slow-draining check valve (CV) emitters.

- The drip line shall feature pressure-compensating emitters.

- The drip line shall feature an anti-siphon feature.

- The drip line shall feature color-coded stripes indicating flow.

- The drip line shall be UV resistant.

- The COP dripline shall be distinguished from other HDL by its copper-red base resin.

- The drip line shall be offered in stretch-wrapped coils.

- The drip line shall feature multiple inlet filters and a wide turbulent labyrinth.

- The drip line shall feature emitters that inhibit debris and root intrusion.

LOC FITTINGS

The fittings shall be compatible with any nominal 16-18mm tubing and dripline.

- The fittings shall be constructed of glass-filled polypropylene.

- The fittings shall feature thread lock connection.

- The fitting shall be reusable.

17 MM BARB FITTINGS

The fittings shall be constructed of Acetal and function with vinyl and PE tubing.

- The fittings shall provide a secure connection.

- The fittings shall feature a dual barb design.

ECO-MAT

The product shall feature a fleece-wrapped dripline and fleece blanket for sub-surface irrigation.

- The product shall have an anti-siphon feature to minimize debris and root intrusion.

- The product shall provide for a 20-40% water savings over standard products.

- The product shall feature non-draining, pressure-compensating emitters that open/close simultaneously.

- The product shall use a porous, non-woven PET (polyethylene Terephthalate) material to move water laterally within the soil.

- The product shall feature a check height of 6’.

ECO-WRAP

The product shall feature fleece-wrapped dripline.

- The product shall have an anti-siphon feature to minimize debris and root intrusion.

- The product shall provide for a 20-40% water savings over standard products.

- The product shall feature non-draining, pressure-compensating emitters that open/close simultaneously.

- The product shall use a porous, non-woven PET (polyethylene Terephthalate) material to move water laterally within the soil.

- The product shall feature a check height of 6’.

SUPPLY TUBING

The tubing shall be constructed of UV-resistant polyethylene.

- The tubing shall be constructed with a thick wall.

- Product dimensions: .700” x .600”

- The tubing shall offer kink resistance.

ECO-INDICATOR

The product shall provide a visual indication of system operation.

- The product shall have a visible bright yellow or reclaimed purple stem indicator.

- The product shall operate when system pressure exceeds 12 PSI (6” version) or 15 PSI (12” version)

- 6” version shall feature a ½” FPT bottom inlet port.

- 12” version shall feature ½” FPT bottom and ½” FPT side inlet port

MLD

The product shall be ¼” dripline for tight spaces and raised planters.

- The product shall offer superior flexibility.

- The product shall provide irrigation without being intrusive to the landscape.

- The product will feature evenly spaced emitters at either 6” or 12” spacing

DISTRIBUTION TUBING

The product shall offer stability and flexibility when using point-source emitters or micro sprays.

- The product shall be constructed of high-quality vinyl or polyethylene for connection to acetal ¼” fittings.

- Product dimensions are .250” x .170”

- The product shall be offered in vinyl for cooler climate installations.

- The product shall be offered in polyethylene for warmer climate installations.

¼” BARBED FITTINGS

The product offers a superior hold with robust acetal construction.

- Acetal construction provides a secure connection.

- Available goof plug lays flat to prevent leaking.

IH RISERS

The product shall offer simple point-to-point and vandal resistant irrigation.

- The product shall be made of flexible PVC.

- The product shall be brown to blend in with landscape.

- The product shall accept any ½” FPT emitter.

- The product shall be pre-assembled.

- The product shall be suitable for at-grade or below-grade installation.

- The product shall be offered in multiple lengths.

- The product shall be available with optional check valve.

- The product shall hold back 12’ of head when specified with check valve.

POINT-SOURCE EMITTERS

The product shall offer accurate irrigation in a wide range of flow rates.

- The product shall feature pressure-compensating for consistent and reliable flow.

- The product shall be color-coded by flow for easy identification.

- The product shall be offered in earth-tone colors that blend in with the surrounding environment.

- The product shall be offered in three inlet variations: ¼” barb, 10-32 thread, ½” FPT.

- The product shall have coined edges for easy grip.

- The product shall have a self-piercing barb.

- The product shall have an optional diffuser cap.

- The product shall have a self-flushing diaphragm.

MULTI-PORT EMITTERS

The product shall offer the ability to irrigate groups of plant effectively from one source.

- The product shall feature six pressure-compensating emitter ports to provide consistent and reliable flow.

- The product shall be color-coded by flow for easy identification.

- The product shall be offered in earth-tone colors to blend in with the surrounding landscape.

- The product shall feature swivel elbows to assist in placing water directly to plant.

- The MPM (Multi-Port-Manifold) shall provide unrestricted flow for each outlet.

MICRO SPRAYS

The product shall apply water accurately for small-area coverage.

SOLO-DRIP

- The product shall feature eight streams of water for thorough coverage.

- The product shall feature an adjustable cap for flow and radius adjustment.

HALO-SPRAY

- The product shall feature and adjustable umbrella of water.

- The product shall feature an adjustable cap for flow and radius adjustment.

TRIO-SPRAY

- The product features full, half, and quarter-circle configurations.

- The product shall feature an adjustable cap for flow and radius adjustment.

RIGID RISERS

The product shall maintain stiffness when used with micro sprays for high-throw applications.

- The product shall accept 10-32 threaded components.

- The product shall be with ½” FPT, ¼” barb or blank inlet configurations.

- The product shall be constructed of HDPE.

MULTI-PURPOSE BOX

The product shall be the ideal size to provide protection and easy access to essential irrigation components.

- The product shall have a small footprint.

- The product shall be available in five colors to blend in with any environment.

- The product shall feature and overlapping lid to prevent debris from entering the box.

- The product shall feature a knock-out bolt hole.

- The product shall feature a UV-protected, non-slip lid.

- The product shall be constructed of HDPE.

AIR/VACUUM RELIEF VALVE

The product shall prevent water hammer and system collapse by discharging air during startup and allowing air to enter during shutdown.

- The product shall release air pockets without premature closure.

- The product shall feature leak-free closure after release.

- The product shall be constructed of UV-protected and corrosion-resistant material.

- The product shall have an operating range up to 80 PSI.

AUTOMATIC FLUSH VALVE

The product shall keep laterals clean by automatically flushing water, air, and debris at each system startup.

- The product shall feature a reversible diaphragm to coordinate with low or high flow.

- The product shall feature a removeable top for diaphragm maintenance.

RZWS

The product shall deliver water across all levels of the root zone for high-efficiency subsurface irrigation of trees and shrubs.

- The product shall feature patented StrataRoot baffles that divert water to all levels of the root zone while adding strength to the unit.

- The product shall feature a durable locking cap for vandal resistance.

- The product shall feature a pressure-compensating bubbler for accurate water flow.

- The product shall feature a built in Hunter swing joint for direct installation to ½” PVC fittings.

- The product shall be pre-assembled.

RZB

The product shall assist in the root watering of small trees and shrubs.

- The product shall feature a solid mesh tube with perforated top.

- The product shall allow oxygen and natural precipitation to reach the root zone.

- The product shall allow for easy installation.