# Pro-Spray® Sprinkler Body Written Specifications

Part 1 – General

  - General Description of Product and Function

The pop-up Hunter Pro-Spray Sprinkler Body is designed for residential and commercial applications in both pressure-regulated and non-pressure-regulated models. Standard Pro-Spray models shall be available with a 2", 3", 4", 6", and 12" (5, 7.5, 10, 15, and 30 cm) pop-up stroke as well as an above-ground shrub head. Pressure-regulated PRS30 and PRS40 models shall be available in a shrub model and a 3", 4", 6", and 12" (7.5, 10, 15, and 30 cm) pop-up stroke. Standard, PRS30, and PRS40 models in 6" and 12" (15 and 30 cm) shall have a side inlet option. Pop-up models shall have the option of a factory- or field-installed drain check valve and a purple body cap for reclaimed water applications. Pressure-regulated models shall have the option of the drain check valve and FloGuard™ Technology to reduce the flow through the riser in the event of a missing nozzle. Factory-installed features shall be designated on the top of the Pro-Spray cap for easy identification.

Part 2 – Parts and Material

  - The Hunter Pro-Spray Sprinkler Body shall be available in the following options:

| Pro-Spray | Pro-Spray PRS30 | Pro-Spray PRS40 |
| --- | --- | --- |
| PROS-00 | PROS-00-PRS30 | PROS-00-PRS40 |
| PROS-00-R | PROS-00-PRS30-R | PROS-00-PRS40-R |
| PROS-02 | PROS-03-PRS30 | PROS-03-PRS40 |
| PROS-03 | PROS-04-PRS30 | PROS-04-PRS40 |
| PROS-04 | PROS-04-PRS30-CV | PROS-04-PRS40-CV |
| PROS-04-CV | PROS-04-PRS30-CV-R | PROS-04-PRS40-CV-R |
| PROS-04-CV-R | PROS-04-PRS30-CV-F | PROS-04-PRS40-CV-F |
| PROS-06 | PROS-04-PRS30-CV-F-R | PROS-04-PRS40-CV-F-R |
| PROS-06-CV | PROS-06-PRS30 | PROS-06-PRS40 |
| PROS-06-CV-R | PROS-06-PRS30-CV | PROS-06-PRS40-CV |
| PROS-12 | PROS-06-PRS30-CV-R | PROS-06-PRS40-CV-R |
| PROS-12-CV | PROS-06-PRS30-CV-F | PROS-06-PRS40-CV-F |
| PROS-12-CV-R | PROS-06-PRS30-CV-F-R | PROS-06-PRS40-CV-F-R |
| PROS-06-SI | PROS-12-PRS30 | PROS-12-PRS40 |
| PROS-12-SI | PROS-12-PRS30-CV | PROS-12-PRS40-CV |
|  | PROS-12-PRS30-CV-R | PROS-12-PRS40-CV-R |
|  | PROS-12-PRS30-CV-F | PROS-12-PRS40-CV-F |
|  | PROS-12-PRS30-CV-F-R | PROS-12-PRS40-CV-F-R |
|  | PROS-06-SI-PRS30 | PROS-06-SI-PRS40 |
|  | PROS-12-SI-PRS30 | PROS-12-SI-PRS40 |

- Specification Descriptions

  - PROS-00 = Shrub

  - PROS-02 = 2" (5 cm) pop-up

  - PROS-03 = 3" (7.5 cm) pop-up

  - PROS-04 = 4" (10 cm) pop-up

  - PROS-06 = 6" (15 cm) pop-up

  - PROS-12 = 12" (30 cm) pop-up

  - -SI = side inlet model for 6" and 12" (15 cm and 30 cm)

  - -CV = includes factory-installed drain check valve, “CHECK VALVE” designated on body cap

  - -PRS30 = pressure-regulated to 30 PSI (2.1 bar; 210 kPa), brown body cap

  - -PRS40 = pressure-regulated to 40 PSI (2.8 bar; 280 kPa), gray body cap

  - -F = includes factory-installed FloGuard technology, “FLOGUARD” designated on body cap

  - -R = includes factory-installed reclaimed water purple cap

- Cap Description

- The Pro-Spray cap shall be made of a non-corrosive, impact-resistant, UV-resistant, heavy-duty ABS material.

- Standard and pressure-regulated models shall be color-coded for field identification.

  - The standard Pro-Spray model shall have a black cap.

  - The Pro-Spray PRS30 model shall have a brown cap with “PRS30” marked on the top.

  - The Pro-Spray PRS40 model shall have a gray cap with “PRS40” marked on the top.

- When specified for reclaimed water applications, the cap will be molded in purple.

- An optional snap-on cap molded in UV-resistant purple acetal shall be available to indicate the use of reclaimed water.

- When specified as factory-installed drain check valve, the cap shall have “CHECK VALVE” marked on the top.

- When specified with the FloGuard feature, the cap shall have “FLOGUARD” marked on the top.

- Seal Description

- The Pro-Spray shall have a pressure-activated, chemical- and chlorine-resistant, UV-stable wiper seal that cleans debris from the riser while it retracts.

- The seal shall be molded around a rigid plastic ring to prevent seal deformation.

- The seal shall prevent the pop-up from sticking in the up position.

- The seal shall seal the riser stem to the cap under normal operating pressures.

- The seal shall be removable from the cap for easy service and be replaceable.

- Body

  - The body of the Pro-Spray shall be constructed of a non-corrosive, impact-resistant, UV-resistant, heavy-duty ABS material.

  - Standard Pro-Spray and PRS30 models in 6" and 12" (15 and 30 cm) shall be available with a ½" FNPT side inlet option.

  - The inlet size shall be ½" thread.

- Riser Description

- The riser of the sprinkler shall be constructed of abrasion- and UV-resistant ABS.

- The riser shall be adjustable for pattern alignment using a two-piece ratchet mechanism.

- The riser shall be compatible with female-threaded nozzles.

- A stainless steel spring shall be assembled with the riser for positive retraction when the irrigation cycle is complete.

- Flush Plug Description

- The Pro-Spray shall have a factory-installed, removable flush plug with a pull-up tab that shall prevent debris from entering the sprinkler during installation.

- The flush plug shall be made of polyethylene.

- Drain Check Valve Description

- The Pro-Spray shall have the option of including either a factory- or field-installed drain check valve.

- The standard Pro-Spray models with drain check valve shall check up to 10' (3 m) in elevation change.

- The pressure-regulated Pro-Spray models with drain check valve shall check up to 14' (4.3 m) in elevation change except for the 3" (7.5 cm) PRS models for field-installed check valve.

  - PROS-03-PRS30 and PROS-03-PRS40 models will hold up to 10' (3 m) of elevation with a field-installed check valve.

- When specified with the drain check valve, the cap shall have “CHECK VALVE” marked on the top.

- FloGuard Technology Description

- Pressure-regulated Pro-Spray models with drain check valve shall include the option of a factory-installed FloGuard Technology feature installed in the riser to reduce flow in the event of a missing nozzle.

  - Pro-Spray Dimensions

- Shrub models

  - PROS-00

  - Height = 1½" (4 cm)

  - Inlet size = ½"

- PROS-00-PRS30 and PROS-00-PRS40

  - Height = 4½" (11 cm)

  - Inlet size = ½"

- PROS-02

  - Retracted height = 4" (10 cm)

  - Pop-up height = 2" (5 cm)

  - Exposed diameter = 2¼" (5.7 cm)

  - Inlet size = ½"

- PROS-03

  - Retracted height = 5" (12.5 cm)

  - Pop-up height = 3" (7.5 cm)

  - Exposed diameter = 2¼" (5.7 cm)

  - Inlet size = ½"

- PROS-04 models

  - Retracted height = 5⅞" (15.5 cm)

  - Pop-up height = 4" (10 cm)

  - Exposed diameter = 2¼" (5.7 cm)

  - Inlet size = ½"

- PROS-06 models

  - Retracted height = 8¾" (22.5 cm)

  - Pop-up height = 6" (15 cm)

  - Exposed diameter = 2¼" (5.7 cm)

  - Inlet size = ½"

- PROS-12 models

  - Retracted height = 16⅛" (41 cm)

  - Pop-up height = 12" (30 cm)

  - Exposed diameter = 2¼" (5.7 cm)

  - Inlet size = ½"

  - Warranty

    - The Pro-Spray Sprinkler Body shall carry a 5-year exchange warranty.

Part 3 – Function and Operation

- Operating Pressure

- The Pro-Spray Sprinkler Body shall operate from 15 to 100 PSI (1.0 to 7.0 bar; 100 to 700 kPa).

- To activate the pressure regulator in PRS30 and PRS40 models, a minimum of 10 PSI (0.7 bar; 70 kPa) differential is required to achieve the desired output pressure.

  - Pressure Regulation

- When specified as PRS30 or PRS40, the sprinkler body shall have a pressure-regulating device as an integral part of the pop-up riser.

- The PRS30 models will prevent fogging or misting of the nozzle spray pattern by maintaining a constant outlet pressure of 30 PSI (2.1 bar; 210 kPa) with inlet pressures of up to 100 PSI (7.0 bar; 700 kPa), regardless of the nozzle installed.

- The PRS40 models will provide optimal distribution uniformity of the Hunter MP Rotator® Nozzle by maintaining a constant outlet pressure of 40 PSI (2.8 bar; 280 kPa) with inlet pressures of up to 100 PSI (7.0 bar; 700 kPa), regardless of the specific MP Rotator Nozzle installed.

  - System Flushing

- The flush plug shall allow the system to be flushed before installing the nozzle.

- The flush plug shall open as the stem extends and close when the stem is in the retracted position, preventing debris from entering the riser.

- The flush plug shall have directional flushing action that allows the water to escape in only one direction.

- When specified with FloGuard Technology, the flush plug shall include a spacer to keep the FloGuard feature deactivated for a complete system flush.

  - Drain Check Valve

- For standard Pro-Spray models, the drain check valve will hold up to 10' (3 m) of elevation.

- For pressure-regulated models, the drain check valve will hold up to 14' (4.3 m) of elevation except for the 3" (7.5 cm) PRS models for field-installed check valve.

  - PROS-03-PRS30 and PROS-03-PRS40 models will hold up to 10' (3 m) of elevation with a field-installed check valve.

- The drain check valve can be factory- or field-installed in pop-up models.

  - FloGuard Technology

- In the event of a missing nozzle, the FloGuard feature shall reduce the flow from the riser to a low-flow 10' (3 m) tall stream indicating the need for maintenance.

© 2022 Hunter Industries™. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.