# I-50 Written Specification

Part 1 – General

1.1  The I-50 is a rotary gear driven sprinkler for commercial applications.  It features an ultra-robust, dirt tolerant gear drive that offers the highest torque output of any rotor in the commercial arena.

Part 2 – Parts and Material

2.1  The I-50 Shall be available in the following versions:  6” Pop-up.

- The sprinkler shall be constructed from corrosion and impact resistant ABS plastic.

- The sprinkler shall be equipped with a durable stainless-steel riser spring.

- The sprinkler shall be available with an optional opposing nozzle housing.

- The sprinkler shall be available with 12 nozzle choices in flow rates from 7.6 to 29.5 GPM conventional and 13.0 to 33.7 GPM ON.

- The sprinkler shall be equipped with a non-strippable gear drive.

- The arc of the sprinkler shall be adjustable from 60-360 degrees.

- The sprinkler shall be equipped with a quick check arc mechanism.

- The sprinkler shall be equipped with a factory installed rubber cover.

- The sprinkler shall be equipped with a standard drain check valve to hold back up to 15’ of elevation.

- The sprinkler shall be available in a reclaimed water ID version.

2.2.  Rotor dimensions

I-50-06

Pop-up height: 6”

Overall height: 10 1/4”

Exposed diameter: 2”

Inlet size: 1” NPT

2.3  Warranty

A.  The sprinkler shall carry a five-year, exchange warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 40 to 100 PSI

A.  Operating pressure range: 40 to 100 PSI

3.2  Radius: 44’ to 69’ conventional, 52’ to 76’ ON

3.3  Flow rate: 7.6 to 29.5 GPM conventional, 13.0 to 33.7 GPM ON

3.4  Precipitation rate: 0.4 in/hr approx.

3.5  Nozzle trajectory: 25 degrees approx.