# Hunter Product Written Specification (PS Ultra)

Part 1 – General

  - General Description of Product and Function

The Hunter PS Ultra is a slim-line spray pop-up, and is available in 2”, 4”, and 6” models. Its sleek but durable body incorporates a reliable two-piece ratchet, sturdy cap, male-threaded riser, and large filter screen, providing double filtration on 4” and 6” models. A check valve is available as a user-installed spare part for PS Ultra models with the large filter screen. Standard PS Ultra models come with a pre-installed Precision Distribution Control™ adjustable nozzle. The full range of nozzle options for the PS Ultra includes the 8A, 10A, 12A, 15A, and 17A, as well as the 5’ x 30’ side strip, which is available on 2” and 4” models only. The flush plug version is also available and comes without a nozzle or filter screen. The PS Ultra comes in a variety of sizes and pre-inserted nozzle options to provide a convenient, effective solution for an array of residential applications.

Part 2 – Parts and Material

- The PS Ultra shall be available in the following options:

| 2 in (5 cm) PSU | 4 in (10 cm) PSU | 6 in (15 cm) PSU |
| --- | --- | --- |
| PSU-02 (flush plug, no large filter) | PSU-04 (flush plug, no large filter) | PSU-06 (flush plug, no large filter) |
| PSU-02-8A | PSU-04-8A | PSU-06-8A |
| PSU-02-10A | PSU-04-10A | PSU-06-10A |
| PSU-02-12A | PSU-04-12A | PSU-06-12A |
| PSU-02-15A | PSU-04-15A | PSU-06-15A |
| PSU-02-17A | PSU-04-17A | PSU-06-17A |
| PSU-02-5SS | PSU-04-5SS |  |

PSU-04 models are available without the large filter screen, specified as PSU-04-NFO (Nozzle Filter Only).

- Cap

- The PS Ultra cap is made of a black, UV resistant, ABS material.

- Body

- The PS Ultra body is made of a black, UV resistant, ABS material.

- Seal

  - The PS Ultra body cap seal is made of an antimicrobial santoprene material mix.

- Riser

- The PS Ultra riser is made of a black, UV resistant, ABS material.

- Large Filter Screen

  - The large filter screen included in 4 in (10 cm) and 6 in (15 cm) pre-inserted nozzle models assembles into the riser.

  - The user-installed check valve assembles into the filter screen.

  - The filter screen is made of a polyethylene material.

- Nozzle Options

- Adjustable nozzles: 8A, 10A, 12A, 15A, 17A

- Fixed arc nozzle: SS30

- Flush plug for easy system flushing

- Check Valve Description

- Hunter p/n 462237 up to 7 ft (3 m) of elevation

- The PS Ultra check valve is made of antimicrobial santoprene and acetal components.

- Check valve assembles into the large filter screen.

  - PS Ultra Dimensions

- PSU-02

  - Retracted height = 5 in (12 cm)

  - Pop-up height = 2 in (5 cm)

  - Exposed diameter = 1.25 in (3 cm)

  - Inlet size = 1/2 in

- PSU-04

  - Retracted height = 7.25 in (18 cm)

  - Pop-up height = 4 in (10 cm)

  - Exposed diameter = 1.25 in (3 cm)

  - Inlet size = 1/2 in

- PSU-02

  - Retracted height = 9.5 in (24 cm)

  - Pop-up height = 6 in (15 cm)

  - Exposed diameter = 1.25 in (3 cm)

  - Inlet size = 1/2 in

  - Warranty

- 2 year warranty period

Part 3 – Function and Operation

  - Operating Pressure

- The recommended operating pressure for the PS Ultra is 20 to 70 psi (1.4 to 4.8 bar; 140 to 480 kPa)

- Radius Options

- Adjustable nozzle radius ranges from 7-19 ft (2.2-5.8 m)

- Fixed arc side strip nozzle covers a 5 x 30 ft (1.5 x 9.1 m) area

- Refer to the catalog or website for radius ranges when using other nozzles on the PS Ultra

  - Flow Rates

  - Precision Distribution ControlTM Adjustable Nozzles

      - 8A flow rate range is approximately 0.18-2.00 GPM (0.72-8.07 l/min)

      - 10A flow rate range is approximately 0.20-2.36 GPM (0.80-9.47 l/min)

      - 12A flow rate range is approximately 0.25-3.36 GPM (0.97-13.43 l/min)

      - 15A flow rate range is approximately 0.39-4.52 GPM (1.51-18.00 l/min)

      - 17A flow rate range is approximately 0.49-5.52 GPM (1.68-21.75 l/min)

  - Fixed Arc Nozzle

    - SS530 flow rate range is approximately 1.10-1.50 GPM (3.5-5.5 l/min)

  - Refer to the catalog or website for flow rate data when installing other nozzles on the PS Ultra.

3.4 Precipitation Rates

- Precision Distribution ControlTM Adjustable Nozzles

  - 8A precipitation rate is approximately 2.65 in/hr (67 mm/hr)

  - 10A precipitation rate is approximately 1.93 in/hr (49 mm/hr)

  - 12A precipitation rate is approximately 1.68 in/hr (44 mm/hr)

  - 15A precipitation rate is approximately 1.59 in/hr (40 mm/hr)

  - 17A precipitation rate is approximately 1.53 in/hr (39 mm/hr)

- Fixed Arc Nozzle

  - SS530 precipitation rate is dependent on the layout

  - Use the Total Area Method to calculate the precipitation rate based on the chosen layout

- Refer to the catalog or website for precipitation rate data when installing other nozzles on the PS Ultra.