# MP Stake Written Specifications

Part 1 – General

  - The MP Stake is a temporary irrigation solution for use with either MP Rotator® nozzles or spray nozzles. Two models come preassembled with a 26" (66 cm) stake, 0.345" distribution tubing, ½" male-threaded connection, and nozzle adapter. A pressure-regulated model comes preassembled with a Hunter Check Valve and pressure-regulated shrub adapter.

Part 2 – Parts and Material

  - The Hunter MP Stake shall be available in the following options:

- MP-STAKE

  - 26" (66 cm) stake, 0.345" tubing to ½" male fitting, PROS-00 shrub adapter

- MP-STAKE-PRS40-CV

  - 26" (66 cm) stake, 0.345" tubing to ½" male fitting, Hunter Check Valve, PROS-00-PRS40 pressure-regulated shrub adapter

  - Specification descriptions

- -PRS40 = Pressure-regulated to 40 PSI (2.8 bar; 280 kPa), gray shrub body identifier

- -CV = Factory-installed Hunter Check Valve

  - Component descriptions

- Stake description

  - The stake and attached adapter shall be made of a black, UV-resistant polybutylene terephthalate and polycarbonate blend.

  - The stake shall be 26" (66 cm) long and preassembled to an adapter for assembly to a nozzle shrub adapter.

- Tubing description

  - The supply tubing shall be made from a flexible, linear, low-density polyethylene.

  - The supply tubing shall have a 0.345" inlet diameter.

  - The supply tubing shall be 36" (91 cm) long.

- Connection fitting description

  - The supply tubing shall be compression fitted to a ½" male-threaded fitting for easy assembly into the lateral line.

  - The fitting shall be made of a ½" male-threaded PVC base and an ABS top.

- Nozzle shrub adapter description

  - The standard MP-STAKE model shall include the black PROS-00 nozzle shrub adapter.

  - The pressure-regulated MP-STAKE-PRS40-CV model shall include the gray PROS-00-PRS40 nozzle shrub adapter.

  - MP Stake dimensions

- MP-STAKE

  - Total height = 28" (71 cm)

  - Inlet = ½" male-threaded

- MP-STAKE-PRS40-CV

  - Total height = 34" (86 cm)

  - Inlet = ½" male-threaded

  - Warranty

- MP Stake models shall have a warranty period of two years.

Part 3 – Function and Operation

- Operating pressure

- The MP Stake shall operate from 30 to 70 PSI (2.1 to 4.8 bar; 210 to 480 kPa).

- Reference the pressure loss charts at hunterindustries.com for the most updated pressure loss performance data.

  - Pressure regulation

- When PRS40 is specified, the staking kit shall include a preassembled, pressure-regulating shrub adapter as an integral part of the kit.

- The PRS40 model will provide optimal distribution uniformity of the MP Rotator by maintaining a constant outlet pressure of 40 PSI (2.8 bar; 280 kPa) when inlet pressures are greater than 10 PSI (0.7 bar; 70 kPa) just below the pressure regulator.

  - Drain check valve

- For the standard MP-STAKE model, no drain check valve shall be included in the staking kit.

- For the pressure-regulated MP-STAKE-PRS40-CV model, the staking kit shall come preassembled with the Hunter Check Valve.

- The Hunter Check Valve has a preset check height of 12' (3.7 m) and adjustability up to 32' (9.8 m) of check height using a flat-head screwdriver from the top of the check valve.

© 2021 Hunter Industries™. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.