Hunter ST-1200-BR Synthetic Turf Rotor Written Specifications

Part 1 – General

1.1  The ST-1200-BR Synthetic Turf Rotor is a long-distance, riser-mounted, gear-driven sprinkler designed for wash-down watering applications.

Part 2 – Parts and Material

2.1  The ST-1200-BR shall be available in the following features

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall have a ratcheting, ABS plastic riser.

- The sprinkler shall be available with five nozzle choices in flow rates from 27.0 to 131.0 GPM (6.13 to 29.76 m3/hr; 102.2 to 495.9 l/min).

- The sprinkler shall have a ratcheting mechanism within the gear drive that allows full rotational movement of the nozzle turret assembly without damage to the sprinkler.

- The arc of the sprinkler shall be adjustable from 40° to 360°.

- The sprinkler shall be adjustable in all phases of installation (i.e., before installation, after installation while static, and after installation while in operation). The sprinkler shall come with a preinstalled #12 nozzle.

- Additional nozzles shall come prepackaged in the box along with the rotor.

2.2.  Rotor dimensions

A. ST-1200-BR

    - Overall height: 12" (30 cm)

    - Overall length: 12" (30 cm)

    - Inlet size: 1½" (40 mm) female BSP

2.3  Warranty

A.  The sprinkler shall carry a five-year component part warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 100 to 120 PSI (2.0 to 6.0 bar; 200 to 600 kPa)

A.  Operating pressure range: 100 to 120 PSI (2.0 to 6.0 bar; 200 to 600 kPa)

3.2  Radius: 67' to 115' (20.4 to 35.1 m)

3.3  Flow rate: 27.0 to 131.0 GPM (6.13 to 29.76 m3/hr; 102.2 to 495.9 l/min)

3.4  Precipitation rate: 1.40 in/hr (35.5 mm/hr), approximately

3.5  Nozzle trajectory: Standard: 22.5°, approximately

© 2021 Hunter Industries™. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.