Hunter ST-1600-HS-B Synthetic Turf Rotor Written Specifications

Part 1 – General

1.1 The ST-1600-HS-B Synthetic Turf Rotor shall be of the gear-driven, rotary type, capable of covering a 107' to 165' (32.5 to 50.3 m) radius at 60 to 120 PSI (4.0 to 8.0 bar; 400 to 800 kPa) with a discharge rate of 96.2 to 326.8 GPM (21.8 to 74.2 m3/hr; 364 to 1,237 l/min).

Part 2 – Parts and Material

2.1 The ST-1600-HS-B shall be available in the following versions: 5" (13 cm) pop-up.

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall include within its packaging one preinstalled nozzle and five interchangeable nozzles discharging 96.2 GPM to 326.8 GPM (21.8 to 74.2 m3/hr; 364 to 1237 l/min).

- The sprinkler shall be capable of both full-circle and adjustable part-circle configurations in one sprinkler.

- The adjustable part-circle feature shall be minutely adjustable from 40° to 360°.

- The sprinkler shall be adjustable in all phases of installation (i.e., before installation, after installation while static, and after installation while in operation).

- The sprinkler shall have a ratcheting mechanism within the gear drive that allows full rotational movement of the nozzle turret assembly without damage to the sprinkler.

- The sprinkler shall have a brass adjustment knob that provides the means to change the sprinkler’s speed of rotation.

- The ST-1600-HS-B speed of adjustment range for a 180° arc setting shall be zero (no rotation) up to 65 seconds at 120 PSI (8 bar; 800 kPa).

- The pop-up assembly shall be constructed of solid tubular stainless steel and have a solid brass machined base.

- The pop-up assembly shall have a stainless-steel retraction spring.

- The pop-up assembly shall be protected from external contamination by a telescoping rubberized boot that is sealed at both ends.

- The sprinkler shall have a rubber cover firmly attached to the top of the riser.

- The sprinkler shall have an exposed surface diameter after installation of 14" (36 cm) and have an overall height of 22½" (57 cm).

- The unit shall have a female 2" (50 mm) BSP inlet.

- The sprinkler shall be adaptable to 2" (50 mm) standard PVC piping with an optional adapter fitting.

2.2.  Rotor dimensions

A. ST-1600-HS-B

    - Pop-up height: 5" (13 cm)

    - Overall height: 22½" (57 cm)

    - Exposed diameter: 14" (36 cm)

    - Inlet size: 2" (50 mm) female BSP

2.3  Warranty

A.  The sprinkler shall carry a five-year component part warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 60 to 120 PSI (4.0 to 8.0 bar; 400 to 800 kPa)

A.  Operating pressure range: 60 to 120 PSI (4.0 to 8.0 bar; 400 to 800 kPa)

3.2  Radius: 107' to 165' (32.5 to 50.3 m)

3.3  Flow rate: 96.2 to 326.8 GPM (21.8 to 74.2 m3/hr; 364 to 1,237 l/min)

3.4  Precipitation rate: 2.25 in/hr (60 mm/hr), approximately

3.5  Nozzle trajectory: Standard: 25°, approximately

© 2021 Hunter Industries™. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.