Hunter ST-1600-HS-BR Synthetic Turf Rotor Written Specifications

Part 1 – General

1.1  The ST-1600-HS-BR Synthetic Turf Rotor is a long-range, riser-mounted, gear-driven sprinkler designed to irrigate corrals, pastures, arenas, and synthetic turf.

Part 2 – Parts and Material

2.1  The ST-1600-HS-BR shall be available with the following features

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall be available with one preinstalled nozzle and five interchangeable nozzles in flow rates from 96.2 to 326.8 GPM (2.08 to 74.2 m3/hr; 364 to 1,237 l/min).

- The sprinkler shall have a ratcheting mechanism within the gear drive that allows full rotational movement of the nozzle turret assembly without damage to the sprinkler.

- The sprinkler shall have a brass adjustment knob that provides the means to change the sprinkler’s speed of rotation. The ST-1600-HS-BR speed of adjustment range for a 180° arc setting shall be zero (no rotation) up to 65 seconds at 120 PSI (8 bar; 800 kPa).

- The arc of the sprinkler shall be adjustable from 40° to 360°.

- The sprinkler shall be adjustable in all phases of installation (i.e., before installation, after installation while static, and after installation while in operation). The sprinkler shall come with a preinstalled #20 nozzle.

- Additional nozzles shall come prepackaged in the box along with the rotor.

2.2.  Rotor dimensions

A. ST-1600-HS-BR

    - Overall height: 8¾" (22 cm)

    - Diameter: 8¼" (21 cm)

    - Inlet size: 2" (50 mm) female BSP

2.3  Warranty

A.  The sprinkler shall carry a five-year component part warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 60 to 120 PSI (4.0 to 8.0 bar; 400 to 800 kPa)

A.  Operating pressure range: 60 to 120 PSI (4.0 to 8.0 bar; 400 to 800 kPa)

3.2  Radius: 107' to 165' (32.5 to 50.3 m)

3.3  Flow rate: 96.2 to 326.8 GPM (21.8 to 74.2 m3/hr; 364 to 1237 l/min)

3.4  Precipitation rate: 2.25 in/hr (60 mm/hr), approximately

3.5  Nozzle trajectory: Standard: 25°, approximately

© 2021 Hunter Industries™. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.