ST-90 Written Specifications

Part 1 – General

1.1  The ST-90 is a long-distance rotary gear-driven sprinkler designed for Synthetic Turf sports field appications. It features a jar-top body cap that allows for quick access to the filter screen with no extra pieces, parts, or tools required.

Part 2 – Parts and Material

2.1  The ST-90 shall be available in the following versions: 3" (8 cm) pop-up.

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall have a ratcheting, ABS plastic riser.

- The sprinkler shall be equipped with a durable, stainless steel riser spring.

- The sprinkler shall be available with 2 nozzle choices in flow rates from 74.5 to 92.0 GPM (6.9 to 20.9 m3/hr; 282.0 to 348 l/min).

- The arc of the sprinkler shall be adjustable from 40° to 360°.

- The sprinkler shall be equipped with through-the-top arc adjustment.

- The sprinkler shall be equipped with a QuickCheck™ arc mechanism.

- The sprinkler shall be equipped with a factory-installed green rubber cover.

- The sprinkler shall be available with a user-installed rubber cover kit.

- All secondary nozzles shall come with blocked standard from the factory.

2.2.  Rotor dimensions

A. ST-90

    - Pop-up height: 3" (8 cm)

    - Overall height: 11" (28 cm)

    - Exposed diameter: 3½" (9 cm)

    - Inlet size: 1½" (40 mm) female NPT

2.3  Warranty

A.  The sprinkler shall carry a five-year exchange warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 100 to 120 PSI (6.9 to 8.3 bar; 690 to 830 kPa)

A.  Operating pressure range: 100 to 120 PSI (6.9 to 8.3 bar; 690 to 830 kPa)

3.2  Radius: 103' to 120' (31.4 to 36.6 m)

3.3  Flow rate: 74.5 to 92.0 GPM (6.9 to 20.9 m3/hr; 282 to 348 l/min)

3.4  Precipitation rate: 1.25 in/hr (35 mm/hr) approximately

3.5  Nozzle trajectory: Standard: 22.5° approximately,