Hunter STG-900 Synthetic Turf Rotor Written Specifications

Part 1 – General

1.1  The STG-900 Synthetic Turf Rotor is a long-distance, rotary, gear-driven sprinkler designed for synthetic turf sports field irrigation applications.

Part 2 – Parts and Material

2.1  The STG-900 shall be available in the following versions: 3" (8 cm) pop-up.

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall have a ratcheting, ABS plastic riser.

- The sprinkler shall be equipped with a durable, stainless steel riser spring.

- The sprinkler shall be available with two nozzle choices in flow rates from 74.5 to 92.0 GPM (6.9 to 20.9 m3/hr; 282.0 to 348 l/min).

- The arc of the sprinkler shall be adjustable from 40° to 360°.

- The sprinkler shall be equipped with through-the-top arc adjustment.

- The sprinkler shall be equipped with a QuickCheck™ Arc Mechanism.

- The sprinkler shall be equipped with a factory-installed green rubber cover.

- The sprinkler shall be available with a user-installed Rubber Cover Kit.

- All secondary nozzles shall come blocked standard from the factory.

2.2.  Rotor dimensions

A. STG-900

    - Pop-up height: 3" (8 cm)

    - Overall height: 14" (35.5 cm)

    - Exposed diameter: 8" (20.3 cm)

    - Inlet size: 1½" (40 mm) Acme

2.3  Warranty

A.  The sprinkler shall carry a five-year component part warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 100 to 120 PSI (6.9 to 8.3 bar; 690 to 830 kPa)

A.  Operating pressure range: 100 to 120 PSI (6.9 to 8.3 bar; 690 to 830 kPa)

3.2  Radius: 103' to 120' (31.4 to 36.6 m)

3.3  Flow rate: 74.5 to 92.0 GPM (6.9 to 20.9 m3/hr; 282 to 348 l/min)

3.4  Precipitation rate: 1.25 in/hr (35 mm/hr), approximately

3.5  Nozzle trajectory: Standard: 22.5°, approximately

© 2021 Hunter Industries™. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.